Pamukkale Üniversitesi, Mühendislik Fakültesi
Bilgisayar Mühendisliği Bölümü

Lisans Bitirme Tezi

Elektrik Sistemlerinin 
Uzaktan Denetimi ve Yönetimi: intelliPWR

Veysel Berk ALTUN 
13253004

Danışman

Dr. Öğr. Ü. Elif HAYTAOĞLU

Mayıs 21, 2018

Denizli

Pamukkale Üniversitesi, Mühendislik Fakültesi
Bilgisayar Mühendisliği Bölümü

Lisans Bitirme Tezi

Elektrik Sistemlerinin 
Uzaktan Denetimi ve Yönetimi: intelliPWR

Veysel Berk ALTUN 
13253004

Danışman

Dr. Öğr. Ü. Elif HAYTAOĞLU

Mayıs 21, 2018

Denizli

LİSANS TEZİ ONAY FORMU

Veysel Berk ALTUN tarafından Dr. Öğr. Ü. Elif HAYTAOĞLU yönetiminde hazırlanan “Elektrik Sistemlerinin Uzaktan Denetimi ve Yönetimi: intelliPWR” başlıklı tez tarafımızdan okunmuş, kapsamı ve niteliği açısından bir lisans tezi olarak kabul edilmiştir.

Dr. Öğr. Ü. Elif HAYTAOĞLU
Danışman

| ______________________________ Jüri Üyesi | ______________________________ Jüri Üyesi |
| --- | --- |

Pamukkale Üniversitesi Mühendislik Fakültesi Bilgisayar Mühendisliği Bölüm Kurulu’nun
_____ / _____ / _____ tarih ve __________ sayılı kararıyla onaylanmıştır.

Prof. Dr. Sezai Tokat
Bölüm Başkanı

Bu tezin tasarımı, hazırlanması, yürütülmesi, araştırmalarının yapılması ve bulgularının analizlerinde bilimsel etiğe ve akademik kurallara özenle riayet edildiğini; bu çalışmanın doğrudan birincil ürünü olmayan bulguların, verilerin ve materyallerin bilimsel etiğe uygun olarak kaynak gösterildiğini ve alıntı yapılan çalışmalara atfedildiğine beyan ederim.

Veysel Berk ALTUN
  	Mayıs 2018
 	İmza:

# ÖNSÖZ

Tez çalışmam sırasında kıymetli bilgi, birikim ve tecrübeleri ile bana yol gösterici ve destek olan değerli danışman hocam sayın Dr. Öğr. Ü. Elif HAYTAOĞLU’na, ilgisini ve önerilerini göstermekten kaçınmayan Bilgisayar Mühendisliği Bölümü Ana Bilim Dalı Başkanı sayın Prof. Dr. Sezai Tokat’a sonsuz teşekkür ve saygılarımı sunarım.

Her türlü maddi ve manevi destekleriyle beni hiçbir zaman yalnız bırakmayan Gülyesa ALTUN’a ve aileme sonsuz teşekkür ederim.

Son olarak yardımlarını hiç esirgemeyen değerli arkadaşlarım Gülşah ARAS’a, Osman YILMAZTÜRK’e, Egemen AYHAN’a ve Ahmet YILMAZ’a da teşekkürlerimi bir borç bilirim.

Veysel Berk ALTUN
 	Mayıs 2018

İÇİNDEKİLER

ÖNSÖZ	V

KISALTMALAR	VI

TABLO LİSTESİ	VII

ŞEKİL LİSTESİ	VIII

ÖZET	IX

ABSTRACT	X

1.	GİRİŞ	1

2.	ALTYAPI	2

2.1	Arduino	2

2.2	NodeMCU	2

2.3	openHAB	3

2.4	Mosquitto	4

3.	DONANIM	6

3.1	Arduino Pro Mini	6

3.2	NodeMCU	7

3.3	FOTEK SSR–40DA	8

3.4	MQ–2: Yanıcı Gaz ve MQ–7: Karbonmonoksit Gaz Sensörü	9

3.5	DHT11: Isı Ve Nem Sensörü	9

4.	KÜTÜPHANE	10

4.1	Serializer	10

4.2	MasterScanner	11

4.3	TimerQueue	12

5.	UYGULAMA	14

5.1	Devre Tasarımları	14

5.2	Prototip	15

6.	SONUÇ	18

7.	KAYNAKÇA	19

8.	EKLER	20

8.1	Serializer.CPP	20

8.2	MasterScanner.CPP	26

8.3	TimerQueue.CPP	32

8.4	Master.INO	35

8.5	Slave.INO	55

9.	ÖZGEÇMİŞ	62

# KISALTMALAR

AVR:	Automatic Voltage Regulator
AX: 	Analog X
DX: 	Digital X
DHT: 	Dew, Humidity & Temperature
HAB:	Home Automation Bus
IDE: 	Integrated Development Environment
IO:	Input – Output
IoT: 	Internet of Things
LED: 	Light Emitting Diode
MHz: 	Megahertz
MQ: 	Mĭngăn Qǐ lai (Sensitive to Gas)
MQTT: 	Message Queuing Telemetry Transport
PWM: 	Pulse Width Modulation
RAW: 	Read After Write
R&D:	Research and Development
SoC:	System on A Chip
SPI: 	Serial Peripheral Interface
SSR: 	Solid State Relay
UART: 	Universal Asynchronous Receiver & Transmitter
Wi–Fi: 	Wireless Fidelity
WPA2: 	Wi–Fi Protected Access 2

# TABLO LİSTESİ

Tablo 2.1: 	openHAB Cloud Kurulumu 	 3
Tablo 2.2: 	Mosquitto Kurulumu 	 4

# ŞEKİL LİSTESİ

Şekil 3.1:  	Arduino Pro Mini Pim Diyagramı 	 5
Şekil 3.2: 	NodeMCU Pim Diyagramı 	 6
Şekil 3.3: 	FOTEK SSR–40DA 	 7
Şekil 3.4: 	MQ–2: Yanıcı Gaz ve MQ–7: Karbonmonoksit Gaz Sensörü 	 8
Şekil 3.5: 	DHT11: Isı Ve Nem Sensörü 	 8
Şekil 5.1: 	Master devre tasarımı 	 13
Şekil 5.2: 	Slave devre tasarımı 	 14
Şekil 5.3: 	Örnek Priz Modeli 	 14
Şekil 5.4: 	Örnek Pogo Pimleri 	15
Şekil 5.5: 	Prototip 	15

# ÖZET

Elektriğin ulaştığı her alanda hayata geçirilmesi planlanan bu projede günümüzün priz temelleri yeniden ele alınmış ve tasarımı konusunda inovatif bir fikir ortaya atılmıştır. Yeniden tasarlanan bu prizler ile gelecekte üretilecek olan yeni nesil akıllı ev aygıtlarının bu prizler ile haberleşmesi sağlanmış ve bu akıllı ev aygıtlarının prizler yardımıyla uzaktan denetimi ve yönetimi gerçekleştirilmiştir.

Ortaya atılan bu yeni priz tasarımı fikrinde, priz soketlerinin faz ve nötr pimlerini altına 5 adet küçük bağlantı pimi eklenmiş ve bu pimler yardımı ile prize bağlanan aygıtların geçerli priz ile haberleşmesi amaçlanmıştır.

Tezin Ar–Ge sürecinde kablolu haberleşme süreçlerinde kullanılması amacıyla RaspBerry Pi 3, Arduino Mega, Arduino Pro Mini ve NodeMCU platformları ile uyumlu 3 farklı kütüphane yazılmıştır. Bütün bu kütüphanelerin yazım aşamasında C, C++ ve Phyton dilleri tercih edilmiş; daha kolay bir Ar–Ge süreci için ise GIT versiyon kontrol sistemi kullanılmıştır. Tezin kablosuz haberleşme süreçlerinde ise açık kaynak kodlu olan ve halen geliştirilmeye devam edilen openHAB sistemi tercih edilmiştir.

Sonuç olarak bütün proje kapsamında temel bilgisayar mühendisliği etiğine ve kurallarına uygun olarak fonksiyonel ve modüler toplam 4400 satır kod yazılmış ve çağımıza uygun yeni nesil bir priz tasarımı ortaya konmuştur.

# ABSTRACT

This thesis project is planned to be employed on to every area where the electricity is reached and in this project, today's bases of the sockets reconsidered and an innovative idea have been put forward about the socket's design. With these redesigned sockets, the communication of future generations of intelligent home appliances is provided with sockets and remote control and management of these smart home devices were realized with the help of redesigned sockets.

In this our new socket design include, five small connection pins inserted under the phase and neutral pins of the socket. With the help of these pins, devices connected to the socket are intended to communicate with the current socket.

In the research and development process of the thesis, with the aim of being used in the wired communication processes, three different libraries are written which are compatible with RaspBerry Pi 3, Arduino Mega, Arduino Pro Mini and NodeMCU platforms. In the writing stage of all these libraries, C, C++ and Phyton languages are preferred; the GIT version control system is used for an easier research and development process. In the wireless communication processes of the thesis, openHAB system which is an open source code and which is still being developed is preferred.

As a result, within the scope of the whole project, a total of 4400 lines of functional and modular codes were written in accordance with the basic computer engineering ethics and rules, and a new generation socket design suitable for modernization has been put forward.

# GİRİŞ

Günümüzde insanlığın elektriğe olan ihtiyacını bir bitkinin suya olan ihtiyacı ile benzer şekilde tanımlayabiliriz. Bir bitki su olmadan nasıl en temel faaliyetlerini gerçekleştiremiyorsa, insanlar da artık elektrik olmadan yaşayamıyor ve gündelik faaliyetlerinin çoğunu gerçekleştiremiyorlar. Elektriğin insan hayatında bu derece önemli olduğu 21. Yüzyılda, kullanım alanları ve popülaritesi gün geçtikçe artan “Nesnelerin İnterneti – Iot (Internet of Things)” ile yapılar içerisindeki elektrik kaynaklarına artık uzaktan erişebilmekte ve yönetebilmekteyiz.

İngilizce karşılığı Internet of Things olan Nesnelerin İnterneti, ilk kez 1999 yılında Britanyalı teknoloji öncüsü Kevin Ashton tarafından ortaya atılmış bir terimdir. En genel tanımıyla fiziksel aygıt, sensör, araç ve elektronik aletlerin, sahip oldukları yazılımsal ve donanımsal kaynaklar ile bir ağ üzerinden birbirleri ile haberleşmeleri olarak tanımlanabilir. Benzer biçimde çeşitli haberleşme protokolleri sayesinde birbirleri ile haberleşen ve birbirine bağlanarak akıllı bir ağ oluşturmuş aygıtlar sistemi olarak da tanımlamak mümkündür.

Nesnelerin İnternetine alternatif olarak günümüzde birçok benzer kavramlar vardır; ancak Nesnelerin İnterneti, bu fenomeni açıklamak için en popüler terimdir.

Günümüzde basit bir IoT aygıtının internet ile haberleşmesi aşamasında kullanılan en yaygın yöntem Wi–Fi teknolojisi olarak tanımlanabilir. Herhangi bir üreticinin IoT kavramına uygun bir aygıt üretmeyi planlaması durumunda, Wi–Fi teknolojisi üzerine bir Ar–Ge yapması kaçınılmazdır. Benzer biçimde üretmeyi planladığı yeni nesil bir akıllı ev aygıtına Wi–Fi teknoloji destekleyen bir donanım eklemesi gerekmektedir. Bütün bu süreçler ise bir üreticiye ek bir kaynak tüketimi oluşturur.

Tam da bu noktada, üreticilerin kaynak tüketimini daha aza indirmek amacıyla inovatif bir çözüm arayışı doğmuştur. Literatürde yapılan uzun araştırmalar sonucu, donanımlar arası Wi–Fi haberleşme altyapısını gruplar halinde gerçekleştirecek bir öneri ortaya atılmıştır.

Bu aşama bir IoT aygıtının elektrik olmadan çalışamayacağı varsayılarak, gruplar halinde Wi–Fi haberleşme teknolojisini sağlayacak olan yeni nesil priz tasarımı ortaya atılmıştır. Bu prizler ile üreticilerin Wi–Fi teknoloji ya da benzer herhangi bir haberleşme altyapısına ihtiyaç duymadan, aynı haberleşme sürecinin daha düşük bir bütçe ile gerçekleştirebilmesi amaçlanmıştır.

# ALTYAPI

Tezin uygulama süreçlerinde yapılan testlerin ve uygulamaların tamamında Ubuntu işletim sisteminin 14.04 sürümü tercih edilmiştir. Bu nedenle örnek bir uygulama sürecinde yapılacak olan bütün uygulamaların, bu işletim sisteminin daha eski sürümlerinde yapılacak olan uygulamalar ile aynı sonucu vereceği kesinlikle teyit edilemez.

Bir diğer önemli konu, MQTT haberleşme sürecinde zorunlu olarak gereken aktif internet bağlantısı üzerinedir. Bu aşamada port blocking özelliği olmayan WPA2 Personal kimlik doğrulamalı bir bireysel ağ yapısı tercih edilmiştir. Tezin uygulama süreçlerinin güvenlik üzerine yeterli bir Ar–Ge çalışması içermemesinden dolayı kamuya açık olan ağların bu tez sürecinde kullanılması önerilemez. Tercihen kablolu internet bağlantısı da alternatif olarak kullanılabilir.

Ubuntu işletim sisteminin kurulumu hakkında detaylı bilgi için;
https://www.ubuntu.com/download

## Arduino

Arduino, Giriş/Çıkışları bulunan fiziksel bir programlama platformundan oluşan güncel bir geliştirme kartıdır. Bu geliştirme kartının uygulama ve geliştirme süreçlerinde Processing/Wiring dilinin temel alan bir yazılım kullanılmaktadır.

Arduino kartlarının donanımında bir adet Atmel AVR mikrodenetleyici (ATmega328, ATmega2560 veya ATmega32u4 gibi), bir adet programlayıcı ve diğer devrelere bağlantı için gerekli olan yan devre elemanları bulunur. Her Arduino kartında en az bir adet 5V regüle entegresi ve bir adet 16MHz kristal osilator devresi bulunur. Arduino kartlarını programlamak için harici herhangi bir programlayıcıya ihtiyaç duyulmaz; çünkü bu geliştirme kartının mikro denetleyicisine üretim aşamasında önceden bir bootloader programı yazılmıştır.

Arduino platformundaki bütün uygulama geliştirme süreçlerinde Arduino IDE yazılımının 1.8.5 sürümü tercih edilmiştir.

Arduino IDE yazılımının kurulumu hakkında detaylı bilgi için;
https://www.arduino.cc/en/Main/Software

## NodeMCU

NodeMCU, Giriş/Çıkışları bulunan fiziksel bir programlama platformundan oluşan güncel bir geliştirme kartıdır. Arduino ile benzer yapıya sahip olsa da üzerine gömülü olan ESP8266 Wi–Fi SoC ile kablosuz haberleşme uygulamalarında da kullanılabilir. Sahip olduğu bu farklı donanım yapısından dolayı IoT uygulamalarında yoğun olarak tercih edilmektedir.

NodeMCU platformu her ne kadar Lua Scripting programlama dilini benimsemiş olsa da uygun yapılandırmalar ile NodeMCU platformunun bütün uygulama geliştirme süreçleri Arduino IDE yazılımıyla da yapılabilir.

NodeMCU platformunun Arduino IDE kurulumu hakkında detaylı bilgi için; https://github.com/esp8266/Arduino

## openHAB

openHAB (open Home Automation Bus), Java SE Runtime Environment dilinde yazılmış olup, bina otomasyonunda kullanılan bileşenleri üretici ve iletişim protokolü gözetmeden bir platformda birleştiren yazılım çözümüdür. Bununla birlikte herhangi bir işletim sistemine bağlı olmaksızın ek bağlantılarla yeni teknolojilerle ve protokollerle uygulanabilir.

openHAB yazılımının Ubuntu işletim sisteminde kararlı çalışabilmesi için işletim sistemine yüklü uygun bir Java SE Runtime Environment dağıtımı yazılım gerekmektedir. Buna ek olarak Java SE Runtime Environment yazılımın 1.9.x sürümleri openHAB ile kararlı bir şekilde çalışmadığından, bu yazılımın 1.8.x sürümleri kullanıcılar için resmi olarak önerilmektedir.

Tezin uygulama süreçlerindeki uzak bağlantılar için openHAB Foundation tarafından barındırılan openHAB Cloud altyapısı kullanılmıştır. Bu altyapının, openHAB yazılımı ile kararlı çalışabilmesi için bir takım yapılandırma ayarlarının yapılması gerekmektedir.

| Dosya | Kurulum |
| --- | --- |
| UUID | /var/lib/openhab2/uuid |
| Secret | /var/lib/openhab2/openhabcloud/secret |

Tablo 2.1: openHAB Cloud Kurulumu

openHAB Cloud ile kimlik doğrulaması için yerel openHAB yazılımı, openHAB Cloud servisinin hesap ayarlarınızda girilmesi gereken iki değer üretir. İlki, çalışma zamanınızı tanımlamaya izin veren benzersiz bir kimlik tanımlayıcıdır. İkincisi, şifre olarak hizmet sunan rastgele bir gizli anahtardır. Her iki değer de yerel dosya sistemine yazılır.

Bu dosyaları bir sebepten dolayı hasar görmesi durumunda, openHAB yazılımı otomatik olarak yenilerini oluşturur. Daha sonra UUID ve SECRET değerlerini openHAB Cloud hizmetinde yeniden yapılandırmanız gerekecektir.

openHAB yazılımının kurulumu hakkında detaylı bilgi için; 
https://docs.openhab.org/installation

openHAB Cloud Connector hakkında detaylı bilgi için; https://docs.openhab.org/addons/ios/openhabcloud/readme

MQTT Binding hakkında detaylı bilgi için; https://docs.openhab.org/addons/bindings/mqtt/readme

## Mosquitto

MQTT (Message Queuing Telemetry Transport) protokolü, internette yaygın olarak kullanılan mesaj tabanlı bir haberleşme protokoldür. Hafif bir haberleşme altyapısına sahip oluşu ve düşük kaynak tüketmesiyle IoT ekosistemlerinde sık sık kullanılmaktadır. Hemen hemen tüm IoT bulut platformları akıllı nesnelerden veri gönderip almak için MQTT protokolünü desteklemektedir.

Bu protokolde, istek–yanıt (request–response) yapısına dayalı HTTP'ye karşıt olarak yayın–abone (publish–subscriber) yapısında TCP/IP bağlantısı kurulur. TCP/IP protokolünün çalışabildiği Linux, Windows, Android, iOS ve MacOS gibi bütün işletim sistemleri ile uyumludur. Ayrıca, MQTT protokolü asenkron çalışan bir protokoldür; bu da mesajı beklerken istemci aygıtı engellemediği anlamına gelir. HTTP protokolünün aksine, esas olarak eşzamanlı bir protokoldür. MQTT protokolünün bir başka özelliği, istemcinin ve yayıncının aynı anda internete bağlı olmasını gerektirmemesidir.

Günümüzde IoT platformlarında Mosquitto, Kafka, RabbitMQ, emqttd ve VerneMQ olmak üzere tercih edilebilecek en az 5 adet MQTT yazılımı bulunmaktadır. Bu tezin uygulama süreçlerinde kurulumu ve kullanımı en kolay olan Mosquitto MQTT yazılımı tercih edilmiştir.

openHAB yazılımının bir MQTT istemcisi olarak çalışmasını sağlamak için bir takım yapılandırma ayarlarının yapılması gerekmektedir. Bu yapılandırmalar openHAB yazılımına MQTT işlevselliği kazandırmaz, bunun için Mosquitto veya bir başka altyapı yazılımının kullanılması gerekmektedir.

| Özellik | Varsayılan | Zorunlu |
| --- | --- | --- |
| <broker>.url | – | Evet |
| <broker>.clientId | Rassal | Hayır |
| <broker>.user | – | Evet |
| <broker>.pwd | – | Evet |
| <broker>.qos | 0 | Hayır |
| <broker>.retain | Yanlış | Hayır |
| <broker>.async | Doğru | Hayır |
| <broker>.keepAlive | 60 | Hayır |
| <broker>.allowLongerClientIds | Yanlış | Hayır |

Tablo 2.2: Mosquitto Kurulumu

Bir MQTT yayıncısına mesaj göndermek veya yayınlamak için, services/mqtt.cfg dosyasına kullanılması planlanan tüm aracıların bildirimlerinin yapılması gerekmektedir.

Mosquitto yazılımının kurulumu hakkında detaylı bilgi için; 
https://mosquitto.org/download

# DONANIM

Tezin bütün uygulama süreçlerinde toplamda 2 farklı geliştirme kartı ve 5 farklı devre elemanı kullanılmıştır. Aşağıda bu donanımların özellikleri hakkında detaylı bilgiler verilmiştir.

## Arduino Pro Mini

Arduino Pro Mini, ATmega328 tabanlı bir Arduino geliştirme kartıdır. Bu geliştirme kartı üzerinde 14 adet dijital giriş – çıkış pimi (bu pimlerden 6 tanesi PWM çıkışı), 6 adet analog giriş – çıkış, bir adet kristal osilatör (8 MHz ya da 16 MHz) ve bir adet reset butonu bulundurur. Sahip olduğu entegre voltaj regülatörü ile 12V gerilime kadar voltaj beslemelerine izin vermektedir. Regüle edilmemiş voltaj beslemesi RAW piminden yapılmaktadır.

Arduino Pro Mini geliştirme kartının 3.3V 8 MHz ve 5V 16 MHz olmak üzere iki farklı versiyonu bulunmaktadır.

Şekil 3.1: Arduino Pro Mini Pim Diyagramı

Tezin uygulama süreçlerinde, kablolu haberleşmelerin tamamında I²C protokolü kullanılmıştır. I²C protokolü, UART veya SPI protokollerinden farklı olarak open–drain yapıya sahiptir. Sıradan bir open–drain hattında geçerli pimin çıkış bacağı P–MOSFET ile N–MOSFET devre elemanları arasında bulunmaktadır. open–drain durumuna getirilmiş bir bağlantı hattında MOSFET elemanının source hattı toprağa bağlıdır, gate içeriden sürülmüş durumdadır ve drain açıktadır. Bu aşamada faz ile geçerli bu pim arasında sonsuz empedans oluşur. Bu durumda pimi high konumuna getirebilmek için dışarıdan uygun değerlikte bir pull–up direncinin eklenmesi gerekmektedir. Bu özellikle, çıkışa bağlanacak olan devre elemanının daha fazla akım çekmesi sağlanır ve böylece akım mikroişlemciden değil, pull–up direnci üzerinden çekilir.

Uygulama sürecinde I²C protokolünün bütün SCL ve SDA hatlarına birer adet pull–up direnci bağlanmıştır. Dirençlerin değerleri, yarı iletken üreticisi Microchip Technology’nin ATmega8, ATmega168 ve ATmega328 modelleri için 2016 yılında güncel olarak yayınlamış olduğu kullanım kılavuzundaki formüller ile hesaplanmıştır. Verilen formüller;

fSCL≤100kHz→Rmin=VCC-0.4V3mA , Rmax=1000nsCbus

fSCL>100kHz→Rmin=VCC-0.4V3mA , Rmax=300nsCbus

Formüllerde verilen fSCL değeri SCL hattının çalışma frekansını, Rmin değeri pull–up direncinin sahip olabileceği en küçük direnç değerini, VCC değeri hattın sahip olduğu gerilim değerini, Rmax değeri pull–up direncinin sahip olabileceği en büyük direnç değerini ve son olarak Cbus değeri I²C hattında sahip olunan toplam aygıt sayısını temsil eder.

Formüldeki bütün değişkenlerin uygun değerler ile değiştirilmesi sonucu en uygun pull–up direnç değeri hesaplanabilir. Bu tezin uygulama süreçlerinde kullanılacak olan bütün pull–up direnç değerleri 4.7KΩ olarak baz alınmıştır.

## NodeMCU

NodeMCU, ESP8266–12 tabanlı bir geliştirme kartıdır. Bu geliştirme kartı üzerinde 17 adet dijital giriş – çıkış pimi (bu pimlerden 9 tanesi PWM çıkışı), 1 adet analog giriş – çıkış, bir adet 80 MHz kristal osilatör, bir adet 802.11 b/g/n Wi–Fi modülü ve bir adet reset butonu bulundurur. Sahip olduğu entegre voltaj regülatörü ile 12V gerilime kadar voltaj beslemelerine izin vermektedir.

Şekil 3.2: NodeMCU Pim Diyagramı

## FOTEK SSR–40DA

SSR (Solid State Relay), tamamen elektronik parçalardan oluşturulmuş bir katı hal anahtarlama düzenidir. Klasik röle ve kontaktörle ile aynı işi yapar. Kontaktörler ve röleler gibi kumanda ve güç devrelerine sahiptirler. SSR’lar, elektromekanik rölelere göre daha hızlı çalışmaktadır. Anahtarlama zamanları bir LED’in açılıp kapanma süresine bağlıdır ve bu süre yaklaşık olarak 1ms ile 0.5ms arasındadır. Mekanik bir parçadan oluşmadığı için kullanım ömürleri elektromekanik ve Reed rölelere göre daha uzundur. Elektromekanik ve Reed rölelerin aksine bağlantıları transistörler ile yapıldığı için SSR’ların kontak dirençleri çok daha yüksektir. Ayrıca gelişen teknoloji ile beraber kontak dirençleri sürekli olarak artmaktadır.

Şekil 3.3: FOTEK SSR–40DA

Bu tezin uygulama süreçlerinde FOTEK tarafından geliştirilen SSR–40DA model SSR modülü kullanılmıştır. Giriş voltaj değeri 3~32V ve akım değeri 7.5mA’dir. Çıkış hattında 40A değerine kadar çalışabilen bütün devre elemanlarını sürebilir.

Yukarıda verilen bilgiler ışığında, bu tezde oluşturulan elektrik sistemlerinin kaldırabileceği en yüksek güç değerliğini hesaplayabiliriz. Gücün formülü;

P(W) = I(A) × V(V)

Formüllerde verilen IA değeri akım değerini, V(V) değeri voltaj değerini temsil eder. Formüldeki bütün değişkenlerin uygun değerler ile değiştirilmesi sonucu tez uygulama sürecindeki bir elektrik sisteminin kaldırabileceği maximum güç değerliği hesaplanabilir.

Yapılan hesaplamalar sonucu 220V-50Hz şehir geriliminde bu SSR modülü, ortalama 8880W değerliğine kadar kararlı bir şekilde çalışabilecektir.

## MQ–2: Yanıcı Gaz ve MQ–7: Karbonmonoksit Gaz Sensörü

MQ–2 sensörü, ortamda bulunan ve konsantrasyonu 300~10.000PPM arasında değişen yanıcı gazı algılayan bir sensör modülüdür. –100~500°C arasında çalışabilir ve 5V gerilim üzerinde 150mA akım çeker. Analog çıkışı sayesinde algılanan gaz konsantrasyonu kolayca okunabilir.

MQ–7 sensörü, ortamda bulunan ve konsantrasyonu 10~10,000PPM arasında değişen karbonmonoksiti algılayan bir sensör modülüdür. –10~50°C arasında çalışabilir ve 5V gerilim üzerinde 150mA akım çeker. Analog çıkışı sayesinde algılanan gaz konsantrasyonu kolayca okunabilir.

Şekil 3.4: MQ–2: Yanıcı Gaz ve MQ–7: Karbonmonoksit Gaz Sensörü

## DHT11: Isı Ve Nem Sensörü

Dijital bir sıcaklık ve nem sensör modülüdür. Çevresindeki havayı ölçmek için içerisindeki kapasitif nem sensörünü ve termistörü kullanır. Bu sensörün okunan verilerini dijital çıkış pimine aktarır. Ek olarak bu sensör, her 2 saniyede bir çıkış verir.

Şekil 3.5: DHT11: Isı Ve Nem Sensörü

3~5V gerilim aralığında, maksimum 2.5mA akım değerinde 0~50°C derece sıcaklık için ±2°C hassasiyet ile çalışmaktadır.

# KÜTÜPHANE

Tezin uygulama süreçlerinde C++ dilinde 3 adet kütüphane yazılmıştır. Bu üç kütüphanenin hepsi, haberleşme aşamalarındaki işlem süreçlerini kısaltmak ve kolaylaştırmak amacıyla tezin birçok yazılımsal bölümünde kullanılmıştır.

## Serializer

Serializer kütüphanesi, geliştirme kartları için yazılmış modüler bir çoklu veri paketleme kütüphanesidir. Bu kütüphane, verilen bir veri dizisi önceden belirtilen parametreler ile böler ve çıktı olarak bütün veri dizisinin birleşiminden oluşan tek bir veri dizisi oluşturur. Benzer şekilde bu işlemin tam tersini de gerçekleştirebilir. Kütüphanenin herhangi bir platform da kullanılabilmesi için, projenin ana kod dosyasına ilk olarak Serializer kütüphanesinin bildirilmesi gerekir. Bu adım için uygulanacak olan işlem aşağıdaki gibidir;

// Kütüphaneyi sisteme bildir 
#include <Serializer.h>

Kütüphanede kullanıcıların kullanabileceği 2 adet fonksiyonu vardır. Her iki fonksiyon da parametre olarak sırası ile ayırıcı boyutunu, ayırıcı dizisini, girdi dizisi boyutunu ve girdi dizisini alır. Bu fonksiyonlar;

// Verilen bir serileştirilmiş veriyi çözümle
char **decode(uint16_t sizeofDelim, char delim[], uint16_t sizeofData, char givenData[]);

// Verilen bir serileştirilmemiş veriyi birleştir
char *encode(uint16_t sizeofDelim, char delim[], uint16_t sizeofData, char *givenData[]);

Aşağıda bu kütüphanenin decode() ve encode() fonksiyonları ile Arduino platformunda çalışan örnek bir kullanımı gösterilmiştir;

void setup() {
     // Ayırıcı ve veri listesi 
     char *delim = "_+";
     char *data[] = {"Hello", "World", "Serializer"};

// Birleştir ve sakla
     char *resultofEncode = Serialization.encode(2, delim, 3, data);

// Çıktı –––>
     // "Hello_World+Serializer"
}

Benzer şekilde data verisinin serileştirilmiş çıktısını olan resultofEncode verisini decode() fonksiyonu ile tekrardan sıralı dizi şeklinde çözümleyebiliriz.

void setup() {
     // Çözümleme ve sakla
     char **resultofDecode = Serialization.decode(2, delim, 24, resultofEncode);

// Çıktı –––>
     // {"Hello_World", "Serializer"}
}

## MasterScanner

MasterScanner kütüphanesi, I²C protokolü destekleyen geliştirme kartları için yazılmış bir veri yolu tarayıcısı kütüphanesidir. Bu kütüphane, bir I²C ağındaki Master özelliğine sahip olan bir aygıtın, aynı ağ üzerindeki Slave aygıtlarını taramasını sağlar ve ağda herhangi bir Slave aygıtta değişiklik meydana geldiğinde bu değişikliği kullanıcıya bildirir.

Kütüphanenin herhangi bir platform da kullanılabilmesi için, projenin ana kod dosyasına ilk olarak MasterScanner kütüphanesinin bildirilmesi gerekir. Bu adım için uygulanacak olan işlem aşağıdaki gibidir;

// Kütüphaneyi sisteme bildir 
#include <MasterScanner.h>

Kütüphanede kullanıcıların kullanabileceği 12 adet fonksiyonu vardır. Bu fonksiyonların kullanımı hakkında kısa bir bilgi aşağıda verilmiştir;

// Tarama aralığı, başlangıç adresi ve bitiş adresi değişkenleri
bool setRange(uint16_t intervalMillis, uint8_t startAddress, uint8_t stopAddress);
bool setRange(uint16_t intervalMillis);
bool setRange(uint8_t startAddress, uint8_t stopAddress);

// Başlangıç adresi ve bitiş adresi aralığını sıfırla
void resetRange();

// Köle aygıtları tara
void scanSlaves();

// Köle aygıtlardaki değişiklikleri event–driven olarak dinle
void onConnectedSlaves(void (*pointer)(uint8_t[], byte));
void onDisconnectedSlaves(void (*pointer)(uint8_t[], byte));

// Başlangıç adresini, bitiş adresini veya tarama aralığını döndür
uint8_t getStartAddress();
uint8_t getStopAddress();
uint8_t getIntervalMillis();

// Toplam bağlı aygıt sayısını hesapla
byte getConnectedSlavesCount();

// Herhangi bir adresin dolu olup olmadığını kontrol et
bool isConnected(uint8_t address);

Aşağıda bu kütüphanenin Arduino platformunda çalışan örnek bir kullanımı gösterilmiştir;

void setup() {
     // Arduino aygıtına event–driven tetikleme fonksiyonları bildir
     MasterScanner.onConnectedSlaves(connectedSlaves);
     MasterScanner.onDisconnectedSlaves(disconnectedSlaves);
}

void loop() {
     // I2C veri yolu hattını tara
     MasterScanner.scanSlaves();
}

// I2C veri yoluna herhangi bir yeni aygıt bağlandığında tetiklenir 
void connectedSlaves(uint8_t data[], byte sizeofData) { }

// I2C veri yolundan herhangi bir aygıt kaldırıldığında tetiklenir
void disconnectedSlaves(uint8_t data[], byte sizeofData) { }

Arduino ve benzeri geliştirme kartları genellikle event–driven olmayan yapıda tasarlanmış geliştirme kartlarıdır. Bu geliştirme kartlarında herhangi bir olayı birkaç istisnai durum dışında kendi başınıza tetikleyemezsiniz. Bu nedenle Arduino ve benzeri geliştirme kartlarında herhangi Slave aygıttın gelen verileri takip etmek istiyorsanız, Arduino’nun loop() yapısı içinde sürekli olarak takip etmek istediğiniz Slave aygıtların yazılımsal kontrolünü yapmalısınız.

Yukarıda verilen MasterScanner kütüphanesi, Arduino’nun event–driven olmayan yapısında event–driven yapıda çalışacak şekilde yazılmıştır. Yani bir kullanıcı herhangi bir I²C ağındaki Master özelliğine sahip olan bir aygıta, herhangi bir Slave özelliğine sahip aygıt bağladığında ilgili fonksiyonlar bu yazılımda artık tetiklenebilecektir.

Başka bir açıdan bu kütüphanenin benzeri diğer kütüphanelerden en büyük ayırt edici özelliği, event–driven olarak yazılmış olmasıdır.

## TimerQueue

TimerQueue kütüphanesi, geliştirme kartları için yazılmış kuyruk yapısına sahip bir zamanlayıcı kütüphanesidir. Belli zaman aralıkları ile çalıştırmak istediğiniz fonksiyonları bu kütüphaneye bildirerek sistem tarafından çalıştırılmasını sağlayabilirsiniz. TimerQueue kütüphanesi de MasterScanner kütüphanesine benzer bir şekilde Arduino ve benzeri geliştirme kartlarında event–driven yapıda çalışacak şekilde yazılmıştır.

Kütüphanenin herhangi bir platform da kullanılabilmesi için, projenin ana kod dosyasına ilk olarak TimerQueue kütüphanesinin bildirilmesi gerekir. Bu adım için uygulanacak olan işlem aşağıdaki gibidir;

// Kütüphaneyi sisteme bildir 
#include <TimerQueue.h>

Kütüphanede kullanıcıların kullanabileceği 9 adet fonksiyonu vardır. Bu fonksiyonların kullanımı hakkında kısa bir bilgi aşağıda verilmiştir;

// Kütüphaneye zaman gecikmesi ve durum değerleri ile fonksiyon ekle
bool attach(void (*pointer)(void));
bool attach(void (*pointer)(void), uint16_t intervalMillis);
bool attach(void (*pointer)(void), bool enabledStatus);
bool attach(void (*pointer)(void), uint16_t intervalMillis, bool enabledStatus);

// Kütüphanenin ana fonksiyonunu başlat, dönüye al veya durdur
void start();
void loop();
void stop();

// Kütüphanenin ana fonksiyonuna kayıtlı bir işlemi başlat veya durdur
bool startProcess(void (*pointer)(void));
bool stopProcess(void (*pointer)(void));

Aşağıda bu kütüphanenin Arduino platformunda çalışan örnek bir kullanımı gösterilmiştir;

void setup() {
     // Kütüphaneye 1 saniye aralıklarla çalışan bir fonksiyon ekle
     TimerQueue.attach(listenFunction, (uint8_t)1000);
     // Kütüphanenin ana fonksiyonunu başlat
     TimerQueue.start();
}

void loop() {
     // Kütüphanenin ana fonksiyonunu dönüye al
     TimerQueue.loop();
}

// Kütüphaneye kaydedilen fonksiyonun bildirimi
void listenFunction() { }

Yukarıda verilen örnek kullanımda, TimerQueue kütüphanesine listenFunction() adında bir fonksiyon iliştirilmiştir. Bu işlemin ardından kütüphane çalıştırılmış ve döngüye alınmıştır. Bu noktadan sonra TimerQueue kütüphanesi geliştirme kartının sahip olduğu entegre osilatör ile sürekli olarak zaman hesaplaması yapacak ve sisteme kaydedilen bu fonksiyonu 1’er saniye zaman aralıklarında tetikleyerek çalıştıracaktır.

# UYGULAMA

Uygulama aşamasında bütün prizler Master, prize bağlanan çevre aygıtlar Slave olarak baz alınmıştır. Örnek bir otomasyon sisteminde herhangi bir çevre aygıtı bu prizlere bağlandığında, bu çevre aygıtlarının bütün fonksiyonları ve üretici bilgileri prizlerin veri tabanına yüklenir. Ardından bu veriler Wi–Fi haberleşme altyapısı ile ağın bağlı olduğu MQTT sunucusuna gönderilir. Gönderilen bu veriler bu mesaja abone olan bütün aygıtlara iletilerek örnek bir haberleşme süreci tamamlanmış olur. Yukarı anlatan örnek bir otomasyon sisteminin donanımsal ve yazılımsal detaylı altyapısı aşağıda verilmiştir.

## Devre Tasarımları

Devre tasarımında NodeMCU geliştirme kartı ile FOTEK SSR–40DA elektronik anahtarı kullanılmış, Arduino Pro Mini geliştirme kartı ile DHT11, MQ–2 ve MQ–7 sensörleri kullanılmıştır. Arduino Pro Mini ile geliştirilen çoklu sensör devresi, akıllı priz sistemleri için örnek bir aygıt olarak tasarlanmıştır. Kullanıcılar bu aşamada amaç ve istekleri doğrultusunda farklı türde cihazlar geliştirebilir ve sisteme bağlayabilirler.

Aşağıdaki şemada, I²C protokolünün Master bölümüne ait devre şeması verilmiştir. Bu devre şemasında D1 ve D2 pimleri I²C bağlantılarına, D6 ve D7 pimleri LED bağlantılarına ve son olarak D5 pimi ise SSR elektronik anahtar bağlantısına rezerve edilmiştir. Bu pimlerden herhangi birisinin değiştirilmesi istendiğinde gerek kod kısmında gerekse yazılım kısmında birtakım değişikliklerin yapılması gerekmektedir.

Şekil 5.1: Master devre tasarımı

Verilen devre şemasındaki akıllı prize herhangi bir aygıt bağlandığında, priz ile aygıt arasında ilk olarak handshake işlemi, yani el sıkışma işlemi yapılır. Ardından I²C ağında Master özelliğine sahip olan NodeMCU aygıtı Slave aygıttan sahip olduğu fonksiyonlarının özelliklerini talep eder. Bu talep işlemi ile Slave aygıtı sahip olduğu harici bütün fonksiyonları Serializer kütüphanesi ile serileştirir ve I²C ağı üzerinden Master aygıta yollar. Bütün bu işlem ve süreçler sonucu prize bağlanan Slave aygıtının haberleşme için gerekli olan bütün özellikleri Master aygıtın veri tabanına kaydedilir ve böylece Slave aygıt haberleşme süreci için hazır konuma geçmiş olur.

Bir sonraki adımda Master aygıt, Slave aygıtın sahip olduğu fonksiyonlardan ilgili verileri belirli zaman aralıklarla talep eder ve dönen verileri MQTT altyapısı ile openHAB yazılımına bildirir.

Yukarıda verilen devre şemasından farklı olarak aşağıdaki verilen şema, I²C protokolünün Slave bölümüne ait devre şemasıdır. Bu devre şemasında A0 pimi MQ–2 sensörüne, A1 pimi MQ–7 sensörüne ve son olarak 13 numaralı pim ise DHT11 sensörüne rezerve edilmiştir. Bu pimlerden herhangi birisinin değiştirilmesi istendiğinde gerek kod kısmında gerekse yazılım kısmında birtakım değişikliklerin yapılması gerekmektedir.

Şekil 5.2: Slave devre tasarımı

## Prototip

Uygulama aşamasına her şeyden önce, ortaya atılan yeni priz tasarımı hakkında örnek bir çizim ve 3 boyutlu prototipleme yapılmıştır. Aşağıda, yapılan bu priz tasarımın 4 farklı açıdan sanal bir modeli verilmiştir.

Şekil 5.3: Örnek Priz Modeli

Yapılan bu protiplere günümüzde sıkça kullanılan pogo pim konnektörleri entegre edilmiş ve son prototip modeline ulaşılmıştır.

Şekil 5.4: Örnek Pogo Pimleri

Şekil 5.5: Prototip

# SONUÇ

Tezin sonucunda gelecekte popülaritesi artacak olan ev otomasyonu sistemleri için düşük maliyetli, modüler, geliştirmeye açık ve kişi merkezli akıllı priz prototipleri yapılmış ve uygulamalı olarak test edilmiştir. Yapılan bu prototipler, herhangi bir kitleyi ve bölgeyi baz almak yerine evrensel açıdan herkesi baz alacak şekilde tasarlamıştır.

Diğer IoT sistemlerinden farklı olarak bu priz prototipleri kurulum gerektirmeyen bir yapıya sahiptir. Prizlere herhangi bir yeni nesil aygıt bağlandığında, bu aygıt için geçerli prize aplikasyon ve haberleşme kurulumu yapılması gerekmez; sisteme bağlanan bu yeni nesil akıllı aygıt evinizin veri tabanına eş zamanlı olarak anında kaydedilir.

Tezin hayata geçirilmesi durumunda, günümüzde yeterliliği daha az olan akıllı ev sistemleri yerine yeterliliği ve kullanılabilirliği daha fazla olan akıllı ev sistemleri inşa edilebilecektir. Benzer şekilde ev elektroniğinde yaşanabilecek olası kazalar azaltılabilecek ve yaşanabilecek herhangi bir kaza önceden tahmin edilebilecektir. Sistemin bütünlük felsefesini benimsemesi nedeniyle kullanım karmaşası da azaltılmış olacaktır.

Düşük maaliyetli oluşundan dolayı yeni tasarlanan bu prizler, yapılardaki eski priz sistemleri ile kolaylıkla değiştirilebilecek ve böylece eski yapıdaki priz sistemleri akıllı priz sistemlerine kolaylıkla dönüştürülebilecektir.

# KAYNAKÇA

[1] 	Robert Clemenzi, “Arduino – Examples and Libraries”, 2015. [Online]. Bağlantı:
http://mc–computing.com/Hardware_Platforms/Arduino/Libraries [Erişim: 01/2018]

[2] 	Dan Hirschberg, “Pointers and Memory Allocation”, 2016. [Online]. Bağlantı:
https://www.ics.uci.edu/~dan/class/165/notes/memory [Erişim: 03/2018]

[3] 	Arduino, “Manuals and Curriculum”, 2016. [Online]. Bağlantı:
https://playground.arduino.cc/Main/ArduinoPinCurrentLimitations [Erişim: 11/2017]

[4] 	tutorialspoint, “C library function void *realloc(void *ptr)”, 2018. [Online]. Bağlantı:
https://www.tutorialspoint.com/c_standard_library/c_function_realloc [Erişim: 12/2017]

[5] 	tutorialspoint, “C library function char *strtok(char *str)”, 2018. [Online]. Bağlantı: 
https://www.tutorialspoint.com/c_standard_library/c_function_strtok [Erişim: 02/2018]

[6] 	GitHUB VCS, “open–source electronics prototyping platform”, 2018. [Online]. Bağlantı:
https://github.com/arduino/Arduino [Erişim: 04/2018]

[7] 	Codingstreet, “C INPUT OUTPUT”, 2013. [Online]. Bağlantı:
http://codingstreet.com/c–input–output [Erişim: 12/2017]

[8] 	armMBED, “C Data Types”, 2018. [Online]. Bağlantı:
https://os.mbed.com/handbook/C–Data–Types [Erişim: 01/2018]

[9] 	cplusplus, “strtok: Split string into tokens”, 2015. [Online]. Bağlantı:
http://www.cplusplus.com/reference/cstring/strtok [Erişim: 02/2018]

[10] 	cplusplus, “strchr: Locate first occurrence of character in string”, 2017. [Online]. Bağlantı:
http://www.cplusplus.com/reference/cstring/strchr [Erişim: 01/2018]

[11] 	cplusplus, “isalnum: Check if character is alphanumeric”, 2016. [Online]. Bağlantı:
http://www.cplusplus.com/reference/cctype/isalnum [Erişim: 02/2018]

[12] 	Wikipedia, “Delimiter”, 2010. [Online]. Bağlantı:
https://en.wikipedia.org/wiki/Delimiter [Erişim: 10/2017]

[13] 	Wikipedia, “C0 and C1 control codes”, 2010. [Online]. Bağlantı:
https://en.wikipedia.org/wiki/C0_and_C1_control_codes [Erişim: 03/2018]

[14] 	aivosto, “Control characters in ASCII and Unicode”, 2016. [Online]. Bağlantı:
http://www.aivosto.com/vbtips/control–characters [Erişim: 01/2018]

[15] 	ElectronicWings, “NodeMCU I2C with Arduino IDE”, 2016. [Online]. Bağlantı:
http://www.electronicwings.com/nodemcu/nodemcu–i2c–with–arduino–ide [Erişim: 01/2018]

[16] 	openHAB Foundation, “MQTT Binding”, 2017. [Online]. Bağlantı:
https://docs.openhab.org/addons/bindings/mqtt1/readme [Erişim: 04/2018]

[17]	GitHUB VCS, “The firmware development of intelliPWR”, 2017. [Online]. Bağlantı:
https://github.com/vberkaltun/nodemcu-firmware-development [Erişim: 10/2017]

# EKLER

Aşağıda tez süresince geliştirilen ve kullanılan kod betikleri verilmiştir. Bu kod betikleri ile örnek bir otomasyonu sistemi yapılabilir.

## Serializer.CPP

/*

* Serializer.cpp

* SERIALIZER - 17.04.2018

*

* =============================================================================

*

* The MIT License (MIT)

*

* Copyright (c) 2018 Berk Altun - vberkaltun.com

*

* Permission is hereby granted, free of charge, to any person obtaining a copy

* of this software and associated documentation files (the "Software"), to deal

* in the Software without restriction, including without limitation the rights

* to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

* copies of the Software, and to permit persons to whom the Software is

* furnished to do so, subject to the following conditions:

*

* The above copyright notice and this permission notice shall be included in

* all copies or substantial portions of the Software.

*

* THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

* IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

* FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

* AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

* LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

* OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE

* SOFTWARE.

*

* =============================================================================

*/

#include "Serializer.h"

// -----

unsigned short Serializer::calculateMaximumLineWidth() {

// IMPORTANT NOTICE: In this step, We need to calculate the bigger size of

// one line in given array. When we declare a 2D array, program will allocate

// each line of this array with same line width. Because of this,

// for avoiding of overflow, we must calculate the upper bound all given line

// Store the size of received data at the here

unsigned short sizeofBuffer = strlen(decodedList.givenData);

char *bufferData = (char *) malloc(sizeof (char) * (sizeofBuffer + 1));

strcpy(bufferData, decodedList.givenData);

bufferData[sizeofBuffer] = '\0';

// -----

// Store the size of received data at the here

unsigned short maximumLineWidth = 0;

// Store splitting data at the here

char *tokenizer = strtok(bufferData, decodedList.delimiter);

while (tokenizer != NULL) {

// Store the size of received data at the here

unsigned short sizeofTokenizer = strlen(tokenizer);

if (sizeofTokenizer > maximumLineWidth)

maximumLineWidth = sizeofTokenizer;

tokenizer = strtok(NULL, decodedList.delimiter);

}

// Do not forget, strictly free up memory

free(bufferData);

return maximumLineWidth;

}

void Serializer::clearDecodedList(bool isAllData) {

// IMPORTANT NOTICE: seems so confused, right?

// When you free up a pointer, you can not use it again anymore

// But when you make reassigning NULL to a pointer after the freeing up,

// You can this pointer again very well

free(decodedList.delimiter);

decodedList.delimiter = NULL;

free(decodedList.givenData);

decodedList.givenData = NULL;

if (isAllData) {

for (unsigned short index = 0; index < decodedList.sizeofResultData; index++) {

free(decodedList.resultData[index]);

decodedList.resultData[index] = NULL;

}

free(decodedList.resultData);

decodedList.resultData = NULL;

decodedList.sizeofResultData = 0;

}

// -----

decodedList.sizeofDelimiter = 0;

decodedList.sizeofGivenData = 0;

}

void Serializer::clearEncodedList(bool isAllData) {

// IMPORTANT NOTICE: seems so confused, right?

// When you free up a pointer, you can not use it again anymore

// But when you make reassigning NULL to a pointer after the freeing up,

// You can this pointer again very well

free(encodedList.delimiter);

encodedList.delimiter = NULL;

for (unsigned short index = 0; index < encodedList.sizeofGivenData; index++) {

free(encodedList.givenData[index]);

encodedList.givenData[index] = NULL;

}

free(encodedList.givenData);

encodedList.givenData = NULL;

if (isAllData) {

free(encodedList.resultData);

encodedList.resultData = NULL;

encodedList.sizeofResultData = 0;

}

// -----

encodedList.sizeofDelimiter = 0;

encodedList.sizeofGivenData = 0;

}

void Serializer::fillDecodedList() {

// Store the size of received data at the here

unsigned short maximumLineWidth = calculateMaximumLineWidth();

// Store splitting data at the here

char *tokenizer = strtok(decodedList.givenData, decodedList.delimiter);

while (tokenizer != NULL) {

// Malloc and realloc a sentence, a list of words

if (decodedList.resultData == NULL)

decodedList.resultData = (char **) malloc(sizeof (char *) * (++decodedList.sizeofResultData));

else

decodedList.resultData = (char **) realloc(decodedList.resultData, sizeof (char *) * (++decodedList.sizeofResultData));

// Malloc and realloc a word, a list of characters, after carry it

decodedList.resultData[decodedList.sizeofResultData - 1] = (char *) malloc(sizeof (char) * (maximumLineWidth + 1));

strcpy(decodedList.resultData[decodedList.sizeofResultData - 1], tokenizer);

decodedList.resultData[decodedList.sizeofResultData - 1][maximumLineWidth] = '\0';

tokenizer = strtok(NULL, decodedList.delimiter);

}

// Do not forget, strictly free up memory

free(tokenizer);

// IMPORTANT NOTICE: Probably it is looking unnecessary ...

// But it has a very important role on memory. With this code,

// Program will resize the array and put a NULL parameter to tail

decodedList.resultData = (char **) realloc(decodedList.resultData, sizeof (char *) * (decodedList.sizeofResultData + 1));

decodedList.resultData[decodedList.sizeofResultData] = '\0';

}

void Serializer::fillEncodedList() {

// Store the size of received data at the here

int sizeofAbsolute = encodedList.sizeofDelimiter - encodedList.sizeofGivenData;

// -----

// Increase total count of data size

encodedList.sizeofResultData += encodedList.sizeofDelimiter;

encodedList.resultData = (char *) malloc(sizeof (char) * (encodedList.sizeofResultData + 1));

// Declare an variable for storing done separator

unsigned short checkedDelimiter = 0;

unsigned short currentIndex = 0;

// If absolute value is bigger than 0, add a delimiter to the first index

if (sizeofAbsolute >= 0)

encodedList.resultData[currentIndex++] = encodedList.delimiter[checkedDelimiter++];

for (unsigned short array = 0; array < encodedList.sizeofGivenData; array++) {

// Store the size of received data at the here

unsigned short sizeofBuffer = strlen(encodedList.givenData[array]);

// Get line by line characters and fill result data

for (unsigned short index = 0; index < sizeofBuffer; index++)

encodedList.resultData[currentIndex++] = encodedList.givenData[array][index];

if (checkedDelimiter != encodedList.sizeofDelimiter)

encodedList.resultData[currentIndex++] = encodedList.delimiter[checkedDelimiter++];

}

// Finally add end null character to tail

encodedList.resultData[currentIndex++] = '\0';

}

bool Serializer::decodeData() {

// Declare an variable for storing done separator

unsigned short checkedDelimiter = 0;

for (unsigned short index = 0; index < decodedList.sizeofGivenData; index++) {

// Found status flag, using for to find operate

bool foundFlag = false;

for (unsigned short subIndex = 0; subIndex < decodedList.sizeofDelimiter; subIndex++) {

if (decodedList.delimiter[subIndex] == decodedList.givenData[index]) {

foundFlag = true;

break;

}

}

// Is a delimiter not found in delimiter list, jump to next

if (!foundFlag)

continue;

if (checkedDelimiter >= decodedList.sizeofDelimiter)

return false;

if (decodedList.delimiter[checkedDelimiter++] != decodedList.givenData[index])

return false;

}

if (checkedDelimiter < decodedList.sizeofDelimiter)

return false;

// Arrived final function

fillDecodedList();

return true;

}

bool Serializer::encodeData() {

// IMPORTANT NOTICE: The absolute value always must be 0 or zero

// For example, If size of given data is bigger or smaller than

// the size of delimiters, we can not have enough delimiters for encoding

// For this reason, When ABS(s) of delimiters and data is 0 or 1,

// encoding can be performed very well

if (abs(encodedList.sizeofGivenData - encodedList.sizeofDelimiter) > 1)

return false;

// -----

for (unsigned short array = 0; array < encodedList.sizeofGivenData; array++) {

// Store the size of received data at the here

unsigned short sizeofBuffer = strlen(encodedList.givenData[array]);

// Check that whether given data includes a delimiters or not

for (unsigned short index = 0; index < sizeofBuffer; index++)

for (unsigned short iterator = 0; iterator < encodedList.sizeofDelimiter; iterator++)

if (encodedList.givenData[array][index] == encodedList.delimiter[iterator])

return false;

// Increase total count of data size

encodedList.sizeofResultData += sizeofBuffer;

}

// Arrived final function

fillEncodedList();

return true;

}

// -----

/**

* Constructors

*

* @param -

* @return -

*/

Serializer::Serializer() {

}

// -----

char **Serializer::decode(unsigned short sizeofDelimiter, char delimiter[], unsigned short sizeofGivenData, char givenData[]) {

// Clear last stored data

clearDecodedList(true);

if (delimiter == NULL)

return NULL;

if (givenData == NULL)

return NULL;

if (sizeofDelimiter == 0)

return NULL;

// Store all received data size on lib

decodedList.sizeofDelimiter = sizeofDelimiter;

decodedList.sizeofGivenData = sizeofGivenData;

// -----

decodedList.delimiter = (char *) malloc(sizeof (char) * (sizeofDelimiter + 1));

strcpy(decodedList.delimiter, delimiter);

decodedList.delimiter[sizeofDelimiter] = '\0';

decodedList.givenData = (char *) malloc(sizeof (char) * (sizeofGivenData + 1));

strcpy(decodedList.givenData, givenData);

decodedList.givenData[sizeofGivenData] = '\0';

// -----

bool decodeDataFlag = decodeData();

if (!decodeDataFlag)

clearDecodedList(true);

else

clearDecodedList(false);

return (decodeDataFlag ? decodedList.resultData : NULL);

}

char *Serializer::encode(unsigned short sizeofDelimiter, char delimiter[], unsigned short sizeofGivenData, char *givenData[]) {

// Clear last stored data

clearEncodedList(true);

if (delimiter == NULL)

return NULL;

if (givenData == NULL)

return NULL;

if (sizeofDelimiter == 0)

return NULL;

// Store all received data size on lib

encodedList.sizeofDelimiter = sizeofDelimiter;

encodedList.sizeofGivenData = sizeofGivenData;

// -----

encodedList.delimiter = (char *) malloc(sizeof (char) * (sizeofDelimiter + 1));

strcpy(encodedList.delimiter, delimiter);

encodedList.delimiter[sizeofDelimiter] = '\0';

// IMPORTANT NOTICE: In Clang, there is no way out to copy 2D array

// If you want to do that, first you must allocate main array memory

// After that you must to fill each line of main array

encodedList.givenData = (char **) malloc(sizeof (char *) * (sizeofGivenData + 1));

for (unsigned short index = 0; index < sizeofGivenData; index++) {

// Store the size of received data at the here

unsigned short sizeofArray = strlen(givenData[index]);

encodedList.givenData[index] = (char *) malloc(sizeof (char) * (sizeofArray + 1));

strcpy(encodedList.givenData[index], givenData[index]);

}

encodedList.givenData[sizeofGivenData] = '\0';

bool encodeDataFlag = encodeData();

if (!encodeDataFlag)

clearEncodedList(true);

else

clearEncodedList(false);

return (encodeDataFlag ? encodedList.resultData : NULL);

}

// -----

/**

* Preinstantiate Objects

*

* @param -

* @return -

*/

Serializer Serialization = Serializer();

## MasterScanner.CPP

/*

* MasterScanner.cpp

* MASTER SCANNER - 23.03.2018

*

* =============================================================================

*

* The MIT License (MIT)

*

* Copyright (c) 2018 Berk Altun - vberkaltun.com

*

* Permission is hereby granted, free of charge, to any person obtaining a copy

* of this software and associated documentation files (the "Software"), to deal

* in the Software without restriction, including without limitation the rights

* to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

* copies of the Software, and to permit persons to whom the Software is

* furnished to do so, subject to the following conditions:

*

* The above copyright notice and this permission notice shall be included in

* all copies or substantial portions of the Software.

*

* THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

* IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

* FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

* AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

* LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

* OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE

* SOFTWARE.

*

* =============================================================================

*/

#include "MasterScanner.h"

void Scanner::onTriggeredConnected(uint8_t _array[], byte _count) {

if (_count != 0) {

// don't bother if user hasn't registered a callback

if (!onConnected) {

return;

}

onConnected(_array, _count);

}

}

void Scanner::onTriggeredDisconnected(uint8_t _array[], byte _count) {

if (_count != 0) {

// don't bother if user hasn't registered a callback

if (!onDisconnected) {

return;

}

onDisconnected(_array, _count);

}

}

bool Scanner::checkRange(uint16_t _intervalMillis) {

if (_intervalMillis < DEFAULT_INTERVAL_MILLIS_MIN)

return false;

if (_intervalMillis > DEFAULT_INTERVAL_MILLIS_MAX)

return false;

return true;

}

bool Scanner::checkRange(uint8_t _startAddress, uint8_t _stopAddress) {

if (_startAddress > _stopAddress)

return false;

if (_startAddress < defaultData.startAddress)

return false;

if (_stopAddress > defaultData.stopAddress)

return false;

return true;

}

void Scanner::cleanRange(uint8_t _startAddress, uint8_t _stopAddress) {

for (uint8_t address = _startAddress; address <= _stopAddress; address++)

givenData.connectedSlavesArray[address] = NULL;

}

uint8_t *Scanner::fillArray(uint8_t _array[], uint8_t _address, byte _count) {

if (_array == NULL)

_array = (uint8_t*) malloc(sizeof (uint8_t) * _count);

else

_array = (uint8_t*) realloc(_array, sizeof (uint8_t) * _count);

_array[_count - 1] = _address;

return _array;

}

// -----

/**

* Constructors

*

* @param -

* @return -

*/

Scanner::Scanner() {

}

// -----

/**

* Changes default scanning range with new range, additional changes delay

*

* @param Delay, start and stop address of range

* @return -

*/

bool Scanner::setRange(uint16_t _intervalMillis, uint8_t _startAddress, uint8_t _stopAddress) {

bool setRangeFlag = false;

if (checkRange(_intervalMillis) && checkRange(_startAddress, _stopAddress)) {

this->setRange(_intervalMillis);

this->setRange(_startAddress, _stopAddress);

setRangeFlag = true;

}

return setRangeFlag;

}

/**

* Changes default scanning range with new range

*

* @param Delay range

* @return -

*/

bool Scanner::setRange(uint16_t _intervalMillis) {

bool setRangeFlag = false;

if (checkRange(_intervalMillis)) {

givenData.intervalMillis = _intervalMillis;

setRangeFlag = true;

}

return setRangeFlag;

}

/**

* Changes default scanning range with new range

*

* @param Start and stop address of range

* @return -

*/

bool Scanner::setRange(uint8_t _startAddress, uint8_t _stopAddress) {

bool setRangeFlag = false;

if (checkRange(_startAddress, _stopAddress)) {

if (_startAddress > givenData.startAddress)

this->cleanRange(givenData.startAddress, _startAddress - 1);

if (_stopAddress < givenData.stopAddress)

this->cleanRange(_stopAddress + 1, givenData.stopAddress);

givenData.startAddress = _startAddress;

givenData.stopAddress = _stopAddress;

setRangeFlag = true;

}

return setRangeFlag;

}

/**

* Resets current scanning range with default range

*

* @param Start and stop address of range

* @return -

*/

void Scanner::resetRange() {

this->setRange(defaultData.intervalMillis, defaultData.startAddress, defaultData.stopAddress);

}

// -----

/**

* Scans connected devices with the given range

*

* @param -

* @return -

*/

void Scanner::scanSlaves() {

uint16_t currentMillis = millis();

if (currentMillis - previousMillis >= givenData.intervalMillis) {

// save the last time you blinked the LED

previousMillis = currentMillis;

// -----

uint8_t *currentConnectedSlavesArray = NULL;

uint8_t *currentDisconnectedSlavesArray = NULL;

byte currentConnectedSlavesCount = 0;

byte currentDisconnectedSlavesCount = 0;

// -----

// addresses 0x00 through 0x77

for (uint8_t currentAddress = givenData.startAddress; currentAddress <= givenData.stopAddress; currentAddress++) {

Wire.beginTransmission(currentAddress);

if (Wire.endTransmission() == 0) {

if (givenData.connectedSlavesArray[currentAddress] == NULL) {

givenData.connectedSlavesArray[currentAddress] = currentAddress;

givenData.connectedSlavesCount++;

currentConnectedSlavesArray = this- >fillArray(currentConnectedSlavesArray, currentAddress, ++currentConnectedSlavesCount);

}

} else {

if (givenData.connectedSlavesArray[currentAddress] != NULL) {

givenData.connectedSlavesArray[currentAddress] = NULL;

givenData.connectedSlavesCount--;

currentDisconnectedSlavesArray = this- >fillArray(currentDisconnectedSlavesArray, currentAddress, ++currentDisconnectedSlavesCount);

}

}

}

onTriggeredConnected(currentConnectedSlavesArray, currentConnectedSlavesCount);

onTriggeredDisconnected(currentDisconnectedSlavesArray, currentDisconnectedSlavesCount);

// always free at the end

free(currentConnectedSlavesArray);

free(currentDisconnectedSlavesArray);

}

}

// -----

/**

* Triggers when at least one slave is connected

*

* @param A function that has two parameters of array and count

* @return -

*/

void Scanner::onConnectedSlaves(void (*pointer)(uint8_t[], byte)) {

onConnected = pointer;

}

/**

* Triggers when at least one slave is disconnected

*

* @param A function that has two parameters of array and count

* @return -

*/

void Scanner::onDisconnectedSlaves(void (*pointer)(uint8_t[], byte)) {

onDisconnected = pointer;

}

// -----

/**

* Gets start address of range

*

* @param -

* @return Start address of range

*/

uint8_t Scanner::getStartAddress() {

return givenData.startAddress;

}

/**

* Gets stop address of range

*

* @param -

* @return Stop address of range

*/

uint8_t Scanner::getStopAddress() {

return givenData.stopAddress;

}

/**

* Gets scanning frequency

*

* @param -

* @return Scanning frequency

*/

uint16_t Scanner::getIntervalMillis() {

return givenData.intervalMillis;

}

/**

* Gets connected slaves count

*

* @param -

* @return Connected slaves count

*/

byte Scanner::getConnectedSlavesCount() {

return givenData.connectedSlavesCount;

}

/**

* Checks if specified address is online on bus

*

* @param Address of specified device

* @return Connected or not Connected

*/

bool Scanner::isConnected(uint8_t _address) {

bool isConnectedFlag = false;

// addresses 0x00 through 0x77

for (uint8_t currentAddress = givenData.startAddress; currentAddress <= givenData.stopAddress; currentAddress++) {

if (givenData.connectedSlavesArray[currentAddress] == _address) {

isConnectedFlag = true;

break;

}

}

return isConnectedFlag;

}

// -----

/**

* Preinstantiate Objects

*

* @param -

* @return -

*/

Scanner MasterScanner = Scanner();

## TimerQueue.CPP

/*

* TimerQueue.cpp

* TIMER QUEUE - 28.04.2018

*

* =============================================================================

*

* The MIT License (MIT)

*

* Copyright (c) 2018 Berk Altun - vberkaltun.com

*

* Permission is hereby granted, free of charge, to any person obtaining a copy

* of this software and associated documentation files (the "Software"), to deal

* in the Software without restriction, including without limitation the rights

* to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

* copies of the Software, and to permit persons to whom the Software is

* furnished to do so, subject to the following conditions:

*

* The above copyright notice and this permission notice shall be included in

* all copies or substantial portions of the Software.

*

* THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

* IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

* FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

* AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

* LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

* OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE

* SOFTWARE.

*

* =============================================================================

*/

#include "TimerQueue.h"

bool Timer::isRegistered(void (*pointer)(void)) {

bool isRegisteredFlag = false;

if (timerQueueArray != NULL) {

for (char currentQueueCount = 0; currentQueueCount < timerQueueCount; currentQueueCount++) {

if (pointer == timerQueueArray[currentQueueCount].pointer) {

isRegisteredFlag = true;

break;

}

}

}

return isRegisteredFlag;

}

bool Timer::checkRange(unsigned long intervalMillis) {

if (intervalMillis < DEFAULT_INTERVAL_MILLIS_MIN)

return false;

if (intervalMillis > DEFAULT_INTERVAL_MILLIS_MAX)

return false;

return true;

}

// -----

/**

* Constructors

*

* @param -

* @return -

*/

Timer::Timer() {

}

// -----

bool Timer::attach(void (*pointer)(void)) {

return this->attach(pointer, DEFAULT_INTERVAL_MILLIS, DEFAULT_STATUS);

}

bool Timer::attach(void (*pointer)(void), unsigned long intervalMillis) {

return this->attach(pointer, intervalMillis, DEFAULT_STATUS);

}

bool Timer::attach(void (*pointer)(void), bool enabledStatus) {

return this->attach(pointer, DEFAULT_INTERVAL_MILLIS, enabledStatus);

}

bool Timer::attach(void (*pointer)(void), unsigned long intervalMillis, bool enabledStatus) {

bool attachFlag = false;

if (!isRegistered(pointer) && checkRange(intervalMillis) && ++timerQueueCount < DEFAULT_QUEUE_SIZE) {

timerQueueArray[timerQueueCount - 1].pointer = pointer;

timerQueueArray[timerQueueCount - 1].intervalMillis = intervalMillis;

timerQueueArray[timerQueueCount - 1].previousMillis = millis();

timerQueueArray[timerQueueCount - 1].enabledStatus = enabledStatus;

attachFlag = true;

}

return attachFlag;

}

// -----

void Timer::start() {

for (char currentQueueCount = 0; currentQueueCount < timerQueueCount; currentQueueCount++)

timerQueueArray[currentQueueCount].previousMillis = millis();

timerEnabledStatus = true;

}

void Timer::loop() {

while (timerEnabledStatus && timerQueueArray != NULL) {

for (char currentQueueCount = 0; currentQueueCount < timerQueueCount; currentQueueCount++) {

if (timerQueueArray[currentQueueCount].enabledStatus == true) {

unsigned long currentMillis = millis();

if (currentMillis - timerQueueArray[currentQueueCount].previousMillis >= timerQueueArray[currentQueueCount].intervalMillis) {

// save the last time you blinked the LED

timerQueueArray[currentQueueCount].previousMillis = currentMillis;

timerQueueArray[currentQueueCount].pointer();

// Additional for NodeMCU

yield();

}

}

}

}

}

void Timer::stop() {

timerEnabledStatus = false;

}

// -----

bool Timer::startProcess(void (*pointer)(void)) {

bool startProcessFlag = false;

if (isRegistered(pointer)) {

for (char currentQueueCount = 0; currentQueueCount < timerQueueCount; currentQueueCount++) {

// save the last time you blinked the LED

if (timerQueueArray[currentQueueCount].pointer == pointer) {

timerQueueArray[currentQueueCount].previousMillis = millis();

timerQueueArray[currentQueueCount].enabledStatus = true;

startProcessFlag = true;

break;

}

}

}

return startProcessFlag;

}

bool Timer::stopProcess(void (*pointer)(void)) {

bool stopProcessFlag = false;

if (isRegistered(pointer)) {

for (char currentQueueCount = 0; currentQueueCount < timerQueueCount; currentQueueCount++) {

// save the last time you blinked the LED

if (timerQueueArray[currentQueueCount].pointer == pointer) {

timerQueueArray[currentQueueCount].enabledStatus = false;

stopProcessFlag = true;

break;

}

}

}

return stopProcessFlag;

}

// -----

/**

* Preinstantiate Objects

*

* @param -

* @return -

*/

Timer TimerQueue = Timer();

## Master.INO

#include <Wire.h>

#include <Serializer.h>

#include <QueueList.h>

#include <MasterScanner.h>

#include <TimerQueue.h>

#include <PubSubClient.h>

#include <ESP8266WiFi.h>

#include <ArduinoOTA.h>

// IMPORTANT NOTICE: These all constant is related with your

// MQTT server and WiFi protocol. In additional, at the here, we are

// Using cloudMQTT server for communication

// IMPORTANT NOTICE: These all constant is depending on your protocol

// As you can see, this protocol delimiter was declared in this scope

// That's mean, all function will use this delimiter constant on

// Communication between two or more devices

#define DEVICE_BRAND "intelliPWR Incorporated"

#define DEVICE_MODEL "MasterCore.X1"

#define DEVICE_VERSION "VER 1.0.0"

// // IMPORTANT NOTICE: We must redeclare our bus range because of

// Subslave of slave device. At the here, we do not need to scan all

// These device on the bus. In any case, slave device will scan their

// Own slave device on the bus

#define I2C_BUS_SDA 5

#define I2C_BUS_SCL 4

#define I2C_START_ADDRESS 0x20

#define I2C_STOP_ADDRESS 0x65

// IMPORTANT NOTICE: On I2C bus, You can send up to 32 bits on

// Each transmission. Therefore, if there is more data than 32 bits

// We should split it. Then we can send our data to master

#define DIVISOR_NUMBER 25

// IMPORTANT NOTICE: Based on buffer size of I2C bus and maximum

// Range of your device. At the here, we declared it with 32 bit

// Because of buffer size of Arduino but if you have a bigger buffer

// Than 32 bit. you can upgrade and speed up your buffers

#define BUFFER_SIZE 32

// IMPORTANT NOTICE: IMPORTANT NOTICE: If buffer size is not looking

// Enough for you, you can extend or shrink your data with this variable.

// Due to lack of resources on memory, we were setted it as 8 but if you

// Have more memory on your device, bigger value can be compatible

#define MINIMIZED_BUFFER_SIZE 16

// Outside protocol delimiters

#define PROTOCOL_DELIMITERS ""

#define PROTOCOL_DELIMITERS_SIZE 3

// Inside protocol delimiters, we called it data delimiters

#define DATA_DELIMITER ""

#define DATA_DELIMITER_SIZE 1

// Start and end type of protocol delimiters

#define IDLE_SINGLE_START 0x15

#define IDLE_MULTI_START 0x16

#define IDLE_MULTI_END 0x17

// -----

enum handshakeData {Unknown, Ready};

enum communicationData {Idle, Continue, End};

enum notifyData {Online, Offline, Confirmed, Unconfirmed};

struct vendorData {

char Brand[BUFFER_SIZE];

char Model[BUFFER_SIZE];

char Version[BUFFER_SIZE];

};

struct functionData {

char Name[BUFFER_SIZE];

bool Request = false;

unsigned short Listen = false;

};

struct deviceData {

struct vendorData vendorList;

QueueList<functionData> functionList;

enum handshakeData handshake = Unknown;

char address = NULL;

};

QueueList<deviceData> deviceList;

enum communicationData communicationFlag = Idle;

enum notifyData notifyFlag = Offline;

// -----

// Do not change default value of this variable

char receivedBuffer[BUFFER_SIZE * BUFFER_SIZE * MINIMIZED_BUFFER_SIZE];

unsigned short sizeofReceivedBuffer = 0;

// Do not change default value of this variable

char givenBuffer[MINIMIZED_BUFFER_SIZE][BUFFER_SIZE];

unsigned short sizeofGivenBuffer = 0;

unsigned short indexofGivenBuffer = 0;

char sizeofFunctionList = 2;

char *functionList[] = {"getVendors",

"getFunctionList"

};

// -----

// MQTT client objects, required for MQTT

void callBack(char* topic, byte* payload, unsigned int length);

WiFiClient wifiClient;

PubSubClient mqttClient(MQTT_SERVER, MQTT_PORT, callBack, wifiClient);

// TEMPORARILY, will be delete on release

#define WIRE_BEGIN 0x01

#define BLINK_FREQUENCY 1000

#define BLINK_RANGE 200

#define BLINK_R D6

#define BLINK_GB D7

#define SSR D5

void setup() {

// Set PWM frequency 500, default is 1000

analogWriteFreq(BLINK_FREQUENCY);

// Set range 0 ~ 100, default is 0 ~ 1023

analogWriteRange(BLINK_RANGE);

// Initialize RGB port and device listen port

pinMode(BLINK_R, OUTPUT);

pinMode(BLINK_GB, OUTPUT);

pinMode(SSR, OUTPUT);

// Reset pins

analogWrite(BLINK_R, BLINK_RANGE);

analogWrite(BLINK_GB, BLINK_RANGE);

digitalWrite(SSR, HIGH);

// -----

// Initialize communication on wire and serial protocol

Serial.begin(9600);

// Initialize communication on Wire protocol

Wire.begin(I2C_BUS_SDA, I2C_BUS_SCL);

Wire.begin(WIRE_BEGIN);

// A name when discovering it as a port in ARDUINO IDE

ArduinoOTA.setHostname(DEVICE_MODEL);

ArduinoOTA.begin();

// Start wifi subsystem

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

connectWiFi();

// -----

// IMPORTANT NOTICE: Due to our I2C scanner lib, When a new device

// Connected or disconnected to master, our I2C scanner lib decides

// Which one is to be triggered

MasterScanner.setRange(I2C_START_ADDRESS, I2C_STOP_ADDRESS);

MasterScanner.onConnectedSlaves(connectedSlaves);

MasterScanner.onDisconnectedSlaves(disconnectedSlaves);

// Attach functions to lib and after run main lib

TimerQueue.attach(listenSlave, (unsigned long)1000);

TimerQueue.attach(listenFunction, (unsigned long)10);

TimerQueue.attach(listenWiFi, (unsigned long)20);

TimerQueue.start();

}

void loop() {

// IMPORTANT NOTICE: As you can see, we do not use delay function

// In all lib. Delay function is a non-blocking function in Arduino

// Core. So solving this, we are using MILLIS()

TimerQueue.loop();

}

void listenSlave() {

MasterScanner.scanSlaves();

}

void listenFunction() {

// IMPORTANT NOTICE: Device registering is more priority than others

// Step, When new device(s) were connected to master device, firstly

// Register these device(s) to system, after continue what you do

for (unsigned short index = 0; index < deviceList.size(); index++) {

// If current index is empty, go next

if (deviceList[index].functionList.size() == 0)

continue;

// If handshake is not ok, that's mean probably function is also not ok

if (deviceList[index].handshake != Ready)

continue;

// Listen functions on connected device(s)

for (unsigned short subindex = 0; subindex < deviceList[index].functionList.size(); subindex++) {

if (!deviceList[index].functionList[subindex].Listen)

continue;

// Store func name at here, we can not use it directly

char internalData1[BUFFER_SIZE];

sprintf(internalData1, "%s", deviceList[index].functionList[subindex].Name);

char *encodeDelimiter1 = generateDelimiterBuffer(DATA_DELIMITER, 1);

char *resultBuffer1[] = {internalData1, "NULL"};

char *result1 = Serialization.encode(1, encodeDelimiter1, 2, resultBuffer1);

// Write internal function list to connected device, one-by-one

writeData(deviceList[index].address, result1);

// We will do this till decoding will return false

while (true) {

// More info was given at inside of this function

// Actually, that's not worst case

if (!readData(deviceList[index].address))

break;

// IMPORTANT NOTICE: At the here, We are making output control.

// The code that given at above changes global flag output(s). So,

// For next operations, We need to check this output(s) and in this

// Way detect our current status

if (communicationFlag != Continue)

break;

}

// If it is still IDLE, that's mean data is corrupted (Not END)

if (communicationFlag == Idle) {

unknownEvent(sizeofReceivedBuffer, receivedBuffer);

break;

}

// -----

// Decode last given data from slave, after we will publish it

char *subDelimiter = generateDelimiterBuffer(DATA_DELIMITER, 1);

char **subBuffer = Serialization.decode(1, subDelimiter, sizeofReceivedBuffer, receivedBuffer);

// Null operator check

if (subBuffer == NULL)

break;

// -----

// Store func name at here, we can not use it directly

char internalData2[BUFFER_SIZE];

sprintf(internalData2, "0x%2x", deviceList[index].address);

// Looking good, inline if-loop

char *encodeDelimiter2 = generateDelimiterBuffer("/", 2);

char *resultBuffer2[] = {internalData2, internalData1};

char *result2 = Serialization.encode(2, encodeDelimiter2, 2, resultBuffer2);

// Publish last received buffer to MQTT broker

mqttClient.publish(result2, subBuffer[1]);

}

}

}

void listenWiFi() {

// Reconnect if connection is lost

if (!mqttClient.connected() && WiFi.status() == 3)

connectWiFi();

// Maintain MQTT connection

mqttClient.loop();

// Allow Wi-Fi functions to run

ArduinoOTA.handle();

}

// -----

void connectedSlaves(uint8_t data[], byte sizeofData) {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

for (char index = 0; index < sizeofGivenBuffer; index++)

for (char subindex = 0; subindex < BUFFER_SIZE; subindex++)

givenBuffer[index][subindex] = '\0';

sizeofGivenBuffer = 0;

indexofGivenBuffer = 0;

// Free up out-of-date buffer data

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

// -----

Serial.print("Done! ");

Serial.print(sizeofData, DEC);

Serial.println(" slave device(s) connected to I2C bus. ID(s): ");

for (unsigned short index = 0; index < sizeofData; index++) {

Serial.print("\t ID: 0x");

Serial.println(data[index], HEX);

}

Serial.println("\t ---");

// Notify end user, status is device online

notifyBlink(0, Online);

// -----

// IMPORTANT NOTICE: Device registering is more priority than others

// Step, When new device(s) were connected to master device, firstly

// Register these device(s) to system, after continue what you do

for (unsigned short index = 0; index < sizeofData; index++) {

// Register last added device to system

for (unsigned short subindex = 0; subindex < sizeofFunctionList; subindex++) {

char *encodeDelimiter = generateDelimiterBuffer(DATA_DELIMITER, 1);

char *resultBuffer[] = {functionList[subindex], "NULL"};

char *result = Serialization.encode(1, encodeDelimiter, 2, resultBuffer);

// Write internal function list to connected device, one-by-one

writeData(data[index], result);

// We will do this till decoding will return false

while (true) {

// More info was given at inside of this function

// Actually, that's not worst case

if (!readData(data[index]))

break;

if (communicationFlag != Continue)

break;

}

if (communicationFlag == Idle) {

unknownEvent(sizeofReceivedBuffer, receivedBuffer);

break;

}

if (!fillConfigurationData(subindex, data[index]))

break;

// -----

// Subscribe all function of current slave to MQTT broker

subscribeTopic(data[index], true);

}

}

}

void disconnectedSlaves(uint8_t data[], byte sizeofData) {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

for (char index = 0; index < sizeofGivenBuffer; index++)

for (char subindex = 0; subindex < BUFFER_SIZE; subindex++)

givenBuffer[index][subindex] = '\0';

sizeofGivenBuffer = 0;

indexofGivenBuffer = 0;

// Free up out-of-date buffer data

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

// -----

Serial.print("Done! ");

Serial.print(sizeofData, DEC);

Serial.println(" slave device(s) disconnected from I2C bus.");

for (unsigned short index = 0; index < sizeofData; index++) {

Serial.print("\t ID: 0x");

Serial.println(data[index], HEX);

// Unsubscribe all function of current slave to MQTT broker

subscribeTopic(data[index], false);

}

Serial.println("\t ---");

// Notify end user, status is device online

notifyBlink(0, Offline);

// -----

// IMPORTANT NOTICE: At the here, firstly, we will make a search about

// Disconnected device. When we find it, We will delete index of this

// Device, and after we will add all temporarily popped device again

// If disconnected count is equal to size of device data, delete all

if (sizeofData >= deviceList.size())

deviceList.clear();

else {

for (unsigned short index = 0; index < sizeofData; index++) {

// Just to be on the safe side, check that queue is empty or not

if (deviceList.size() == 0)

break;

if (data[index] == deviceList[0].address) {

deviceList.popFront();

continue;

}

if (data[index] == deviceList[deviceList.size() - 1].address) {

deviceList.popBack();

continue;

}

for (unsigned short subindex = 1; subindex < deviceList.size() - 1; subindex++) {

if (deviceList[subindex].address == data[index]) {

copyConfigurationData(0, subindex);

deviceList.popFront();

// We found, stop the current session

break;

}

}

}

}

}

// -----

void unknownEvent(unsigned short sizeofData, char data[]) {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

for (char index = 0; index < sizeofGivenBuffer; index++)

for (char subindex = 0; subindex < BUFFER_SIZE; subindex++)

givenBuffer[index][subindex] = '\0';

sizeofGivenBuffer = 0;

indexofGivenBuffer = 0;

// Free up out-of-date buffer data

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

// -----

// Notify user

Serial.print("Error! Unexpected <");

Serial.print(data);

Serial.print(">[");

Serial.print(sizeofData);

Serial.println("] data received.");

// At the end, free up out-of-date buffer data

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

}

bool subscribeTopic(char address, bool type) {

// Make sure we are connected to WIFI before attemping to reconnect to MQTT

if (WiFi.status() != WL_CONNECTED)

return false;

// Make sure we are connected to the MQTT server

if (!mqttClient.connected())

return false;

// -----

// When we found given device in device list, generate MQTT vendor(s)

for (unsigned short index = 0; index < deviceList.size(); index++) {

if (address == deviceList[index].address) {

// Store func name at here, we can not use it directly

char internalData[BUFFER_SIZE];

sprintf(internalData, "0x%2x", address);

// Looking good, inline if-loop

char *encodeDelimiter1 = generateDelimiterBuffer("/", 2);

char *resultBuffer1[] = {internalData, "isConnected"};

char *result1 = Serialization.encode(2, encodeDelimiter1, 2, resultBuffer1);

type ? mqttClient.subscribe(result1) : mqttClient.unsubscribe(result1);

mqttClient.publish(result1, type ? "1" : "0");

char *encodeDelimiter2 = generateDelimiterBuffer("/", 2);

char *resultBuffer2[] = {internalData, "brand"};

char *result2 = Serialization.encode(2, encodeDelimiter2, 2, resultBuffer2);

type ? mqttClient.publish(result2, deviceList[index].vendorList.Brand) : NULL;

char *encodeDelimiter3 = generateDelimiterBuffer("/", 2);

char *resultBuffer3[] = {internalData, "model"};

char *result3 = Serialization.encode(2, encodeDelimiter3, 2, resultBuffer3);

type ? mqttClient.publish(result3, deviceList[index].vendorList.Model) : NULL;

char *encodeDelimiter4 = generateDelimiterBuffer("/", 2);

char *resultBuffer4[] = {internalData, "version"};

char *result4 = Serialization.encode(2, encodeDelimiter4, 2, resultBuffer4);

type ? mqttClient.publish(result4, deviceList[index].vendorList.Version) : NULL;

// -----

for (unsigned short subindex = 0; subindex < deviceList[index].functionList.size(); subindex++) {

if (!deviceList[index].functionList[subindex].Request)

continue;

// Store func name at here, we can not use it directly

char subInternalData0[BUFFER_SIZE * MINIMIZED_BUFFER_SIZE];

sprintf(subInternalData0, "0x%2x", deviceList[index].address);

// Store address at here, we can not use it directly

char subInternalData1[BUFFER_SIZE * MINIMIZED_BUFFER_SIZE];

sprintf(subInternalData1, "%s", deviceList[index].functionList[subindex].Name);

char *subEncodeDelimiter = generateDelimiterBuffer("/", 2);

char *subResultBuffer[] = {subInternalData0, subInternalData1};

char *subResult = Serialization.encode(2, subEncodeDelimiter, 2, subResultBuffer);

// Looking good, inline if-loop

type ? mqttClient.subscribe(subResult) : mqttClient.unsubscribe(subResult);

}

// Do not need to search all data, we are OK now

break;

}

}

}

void callBack(char* topic, byte * payload, unsigned int length) {

// Convert topic to string to make it easier to work with

String stringofTopic = topic;

unsigned short sizeofTopic = stringofTopic.length();

// Print out some debugging info

Serial.print("Done! Callback updated on <");

Serial.print(stringofTopic);

Serial.print(">[");

Serial.print(length);

Serial.println("].");

// Decode payload to char array

char payloadData[length + 1];

for (unsigned short subindex = 0; subindex < sizeofTopic; subindex++)

payloadData[subindex] = (char)payload[subindex];

payloadData[length] = '\0';

// -----

// Generate a delimiter data and use in with decoding function

char *decodeDelimiter = generateDelimiterBuffer("/", 2);

char **decodeBuffer = Serialization.decode(2, decodeDelimiter, sizeofTopic, topic);

// Null operator check

if (decodeBuffer != NULL) {

char internalData0[BUFFER_SIZE];

sprintf(internalData0, "%s", decodeBuffer[0]);

unsigned short sizeofInternalData0 = strlen(internalData0);

char internalData1[BUFFER_SIZE];

sprintf(internalData1, "%s", decodeBuffer[1]);

unsigned short sizeofInternalData1 = strlen(internalData1);

// -----

// Declare a flag, related with device status

bool deviceModelFlag = true;

// Compare internal data(s) with device model, higher priority

for (unsigned short index = 0; index < strlen(DEVICE_MODEL); index++) {

if (internalData0[index] != DEVICE_MODEL[index]) {

deviceModelFlag = false;

break;

}

}

// IMPORTANT NOTICE: If we can arrive there, that's mean the payload

// Data is consisted of status data. Otherwise, we can say the payload

// Data is not consisted of status data. Worst case, Go other state

// Of if, that's mean also it is a function data(s)

if (deviceModelFlag && isNumeric(1, payloadData))

digitalWrite(SSR, (bool)(payload[0] - '0'));

// -----

// Compare internal data(s) with registered function list

for (unsigned short index = 0; index < deviceList.size(); index++) {

// Store func name at the here, we can not use it directly

char internalData2[BUFFER_SIZE];

sprintf(internalData2, "0x%2x", deviceList[index].address);

// Check thar is it same or not

for (unsigned short subindex = 0; subindex < sizeofInternalData0; subindex++)

if (internalData0[subindex] != internalData2[subindex])

break;

// -----

char *encodeDelimiter = generateDelimiterBuffer(DATA_DELIMITER, 1);

char *resultBuffer[] = {internalData1, payloadData};

char *result = Serialization.encode(1, encodeDelimiter, 2, resultBuffer);

// Write internal data list to connected device, one-by-one

writeData(deviceList[index].address, result);

}

}

}

void connectWiFi() {

// Attempt to connect to the wifi if connection is lost

if (WiFi.status() != WL_CONNECTED) {

// Notify user

Serial.print("Connecting to <");

Serial.print(WIFI_SSID);

Serial.print("> ...");

// Loop while we wait for connection

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

// Notify user

Serial.println("");

Serial.print("Done! WiFi connection is OK. IP address was given as <");

Serial.print(WiFi.localIP());

Serial.println(">.");

}

// If we are not connected to server, try to connect server

while (!mqttClient.connected()) {

if (mqttClient.connect((char*) DEVICE_MODEL, MQTT_USER, MQTT_PASSWORD)) {

// IMPORTANT NOTICE: First of all, we need to subscribe main device

// We call it like XXXX/status and this broker is related with SSR

// State of device. We will listen something about this and execute it

char *encodeDelimiter = generateDelimiterBuffer("/", 2);

char *resultBuffer[] = {DEVICE_MODEL, "status"};

char *result = Serialization.encode(2, encodeDelimiter, 2, resultBuffer);

mqttClient.subscribe(result);

}

}

}

void notifyBlink(unsigned short port, enum notifyData status) {

switch (status) {

case Online:

for (int index = 0; index < 3; index++) {

for (int subindex = BLINK_RANGE; subindex > 0; subindex--) {

analogWrite(BLINK_R, subindex);

analogWrite(BLINK_GB, subindex);

delay(1);

}

delay(400);

for (int subindex = 0; subindex < BLINK_RANGE; subindex++) {

analogWrite(BLINK_R, subindex);

analogWrite(BLINK_GB, subindex);

delay(2);

}

delay(800);

}

for (int index = BLINK_RANGE; index > 0; index--) {

analogWrite(BLINK_R, index);

analogWrite(BLINK_GB, index);

delay(1);

}

// Update last stored notify flag

notifyFlag = Online;

break;

case Offline:

switch (notifyFlag) {

case Online:

for (int index = 0; index < BLINK_RANGE; index++) {

analogWrite(BLINK_R, index);

analogWrite(BLINK_GB, index);

delay(1);

}

break;

case Confirmed:

for (int index = 0; index < BLINK_RANGE; index++) {

analogWrite(BLINK_GB, index);

delay(1);

}

break;

case Unconfirmed:

for (int index = 0; index < BLINK_RANGE; index++) {

analogWrite(BLINK_R, index);

delay(1);

}

break;

default:

analogWrite(BLINK_R, BLINK_RANGE);

analogWrite(BLINK_GB, BLINK_RANGE);

break;

}

// Update last stored notify flag

notifyFlag = Offline;

break;

case Confirmed:

for (int index = 0; index < BLINK_RANGE; index++) {

analogWrite(BLINK_R, index);

delay(1);

}

// Update last stored notify flag

notifyFlag = Confirmed;

break;

case Unconfirmed:

for (int index = BLINK_RANGE; index > 0; index--) {

analogWrite(BLINK_GB, index);

delay(1);

}

// Update last stored notify flag

notifyFlag = Unconfirmed;

break;

default:

analogWrite(BLINK_R, BLINK_RANGE);

analogWrite(BLINK_GB, BLINK_RANGE);

// Update last stored notify flag

notifyFlag = Offline;

break;

}

}

// -----

bool writeData(char address, char *data) {

// Free up out-of-date buffer data, top level clearing

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

// Encode given data for bus communication

encodeData(strlen(data), data);

// Send all remainder data to newly registered slave

while (indexofGivenBuffer < sizeofGivenBuffer) {

Wire.beginTransmission(address);

Wire.write(givenBuffer[indexofGivenBuffer++]);

Wire.endTransmission();

// Maybe not need, right?

delay(15);

}

// IMPORTANT NOTICE: Due to decoding of slave device, we need to Wait

// A little bit. Otherwise, Master device will request data From slave

// Device too early and slave cannot send it. Additional, when more

// Devices are connected, we need to downgrade delay time. Already,

// It will take the same time during roaming

delay(100);

// If everything goes well, we will arrive here and return true

return true;

}

bool readData(char address) {

unsigned short indexofNewReceivedBuffer = 0;

char newReceivedBuffer[BUFFER_SIZE];

// Request data from slave

Wire.requestFrom(address, BUFFER_SIZE);

while (Wire.available()) {

// Store new received data at the here

char currentBuffer = Wire.read();

newReceivedBuffer[indexofNewReceivedBuffer++] = (char)currentBuffer;

// Don't forget to add end-of-line char to end

if (currentBuffer == (char)PROTOCOL_DELIMITERS[2]) {

newReceivedBuffer[indexofNewReceivedBuffer] = '\0';

break;

}

}

// Maybe not need, right?

delay(15);

// Decode last given data

if (!decodeData(indexofNewReceivedBuffer, newReceivedBuffer))

return false;

// IMPORTANT NOTICE: Actually When we arrived this point, we arrived

// Worst case point even though It was TRUE. If you came there, program will

// Run till communication flag will be END or IDLE type. Otherwise, this

// Point is related with CONTINUE status

return true;

}

// -----

void copyConfigurationData(char fromAddress, char toAddress) {

// Worst case, We do not have another solution

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (deviceList[fromAddress].vendorList.Brand[index] == NULL) {

deviceList[toAddress].vendorList.Brand[index] = '\0';

break;

}

deviceList[toAddress].vendorList.Brand[index] = deviceList[fromAddress].vendorList.Brand[index];

}

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (deviceList[fromAddress].vendorList.Model[index] == NULL) {

deviceList[toAddress].vendorList.Model[index] = '\0';

break;

}

deviceList[toAddress].vendorList.Model[index] = deviceList[fromAddress].vendorList.Model[index];

}

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (deviceList[fromAddress].vendorList.Version[index] == NULL) {

deviceList[toAddress].vendorList.Version[index] = '\0';

break;

}

deviceList[toAddress].vendorList.Version[index] = deviceList[fromAddress].vendorList.Version[index];

}

// -----

// Worst case, We do not have another solution

for (unsigned short index = 0; index < deviceList[fromAddress].functionList.size(); index++) {

for (unsigned short subindex = 0; subindex < BUFFER_SIZE; subindex++) {

if (deviceList[fromAddress].functionList[index].Name[subindex] == NULL) {

deviceList[toAddress].functionList[index].Name[subindex] = '\0';

break;

}

deviceList[toAddress].functionList[index].Name[subindex] = deviceList[fromAddress].functionList[index].Name[subindex];

}

deviceList[toAddress].functionList[index].Request = deviceList[fromAddress].functionList[index].Request;

deviceList[toAddress].functionList[index].Listen = deviceList[fromAddress].functionList[index].Listen;

}

deviceList[toAddress].handshake = deviceList[fromAddress].handshake;

deviceList[toAddress].address = deviceList[fromAddress].address;

}

bool fillConfigurationData(unsigned short index, char address) {

// IMPORTANT NOTICE: Before decoding inside data, we need firstly

// Calculate count of data delimiter. This result gives us that how

// Many different data included in this inside data

unsigned short countofCharacter = howManyCharacter(DATA_DELIMITER, sizeofReceivedBuffer, receivedBuffer);

// Generate a delimiter data and use in with decoding function

char *decodeDelimiter = generateDelimiterBuffer(DATA_DELIMITER, countofCharacter);

char **newReceivedBuffer = Serialization.decode(countofCharacter, decodeDelimiter, sizeofReceivedBuffer, receivedBuffer);

// Null operator check

if (newReceivedBuffer == NULL)

return false;

// IMPORTANT NOTICE: In this program, we have two data type, one of them

// Is VENDOR data, other one is FUNCTION data. Because of this we have two

// Statement in switch case

switch (index) {

case 0:

if (!fillVendorData(address, countofCharacter, newReceivedBuffer))

return false;

// Notify user

Serial.print("Done! The vendors of [0x");

Serial.print(address, HEX);

Serial.println("] address were saved successfully to system.");

Serial.print("\t BRAND: ");

Serial.println(deviceList[0].vendorList.Brand);

Serial.print("\t MODEL: ");

Serial.println(deviceList[0].vendorList.Model);

Serial.print("\t VERSION: ");

Serial.println(deviceList[0].vendorList.Version);

Serial.println("\t ---");

break;

case 1:

if (!fillFunctionData(address, countofCharacter, newReceivedBuffer)) {

// Notify end user, status is device online

notifyBlink(0, Unconfirmed);

return false;

}

// Notify end user, status is device online

notifyBlink(0, Confirmed);

// Notify user

for (unsigned short index = 0; index < deviceList[0].functionList.size(); index++) {

Serial.print("\t FUNCTION: ");

Serial.println(deviceList[0].functionList[index].Name);

}

Serial.println("\t ---");

break;

default:

return false;

}

// If everything goes well, we will arrive here and return true

return true;

}

bool fillVendorData(char address, unsigned short sizeofData, char **data) {

// Worst case, if vendor list size is not equal to default vendor list size

if (sizeofData != 2) {

struct deviceData newDeviceData;

newDeviceData.handshake = Unknown;

newDeviceData.address = address;

// Major code for device list

deviceList.pushFront(newDeviceData);

unknownEvent(sizeofReceivedBuffer, receivedBuffer);

return false;

}

// IMPORTANT NOTICE: When a new device is registered to master,

// We are decoding all vendor Data at the here. When we are doing

// All of these also we need temp variable

struct deviceData newDeviceData;

// Best case, we have it all

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (data[0][index] == NULL) {

newDeviceData.vendorList.Brand[index] = '\0';

break;

}

newDeviceData.vendorList.Brand[index] = data[0][index];

}

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (data[1][index] == NULL) {

newDeviceData.vendorList.Model[index] = '\0';

break;

}

newDeviceData.vendorList.Model[index] = data[1][index];

}

for (unsigned short index = 0; index < BUFFER_SIZE; index++) {

if (data[2][index] == NULL) {

newDeviceData.vendorList.Version[index] = '\0';

break;

}

newDeviceData.vendorList.Version[index] = data[2][index];

}

// -----

// Major code for device list

deviceList.pushFront(newDeviceData);

// If everything goes well, we will arrive here and return true

return true;

}

bool fillFunctionData(char address, unsigned short sizeofData, char **data) {

// Worst case, function list size is equal to char - 1 size

if ((sizeofData + 1) % 3 != 0)

return false;

for (unsigned short index = 0; index < sizeofData + 1; index += 3) {

char internalData0[BUFFER_SIZE];

sprintf(internalData0, "%s", data[index]);

unsigned short sizeofInternalData0 = strlen(internalData0);

if (!isAlphanumeric(sizeofInternalData0, internalData0))

continue;

char internalData1[BUFFER_SIZE];

sprintf(internalData1, "%s", data[index + 1]);

unsigned short sizeofInternalData1 = strlen(internalData1);

if (!isNumeric(sizeofInternalData1, internalData1))

continue;

char internalData2[BUFFER_SIZE];

sprintf(internalData2, "%s", data[index + 2]);

unsigned short sizeofInternalData2 = strlen(internalData2);

if (!isNumeric(sizeofInternalData2, internalData2))

continue;

// -----

// IMPORTANT NOTICE: When a new device is registered to master,

// We are decoding all function Data at the here. When we are doing

// All of these also we need temp variable

struct functionData newFunctionData;

for (unsigned short subindex = 0; subindex < BUFFER_SIZE; subindex++) {

if (internalData0[subindex] == NULL) {

newFunctionData.Name[subindex] = '\0';

break;

}

newFunctionData.Name[subindex] = internalData0[subindex];

}

newFunctionData.Request = (bool)atoi(internalData1);

newFunctionData.Listen = (bool)atoi(internalData2);

// If we can arrive there, we can say function data is ok

deviceList[0].functionList.pushFront(newFunctionData);

}

// Set other important externals data to internals list

deviceList[0].handshake = Ready;

deviceList[0].address = address;

// If everything goes well, we will arrive here and return true

return true;

}

// -----

bool decodeData(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Decode given data, Calculate up-of-date and needed buffer size

char **newReceivedBuffer = Serialization.decode(PROTOCOL_DELIMITERS_SIZE, PROTOCOL_DELIMITERS, sizeofData, data);

// Null operator check

if (newReceivedBuffer == NULL)

return false;

// Null operator check

if (newReceivedBuffer[0] == NULL ¦¦ newReceivedBuffer[1] == NULL)

return false;

// -----

// IMPORTANT NOTICE: As I said before, NodeMCU do not support lots of

// C++ library. This one is one of these, also. At the here, we can not

// Use directly strlen function. For solving, first we need to get inside data,

// After we need to calculate size of the given data

char internalData0[BUFFER_SIZE];

sprintf(internalData0, "%s", newReceivedBuffer[0]);

unsigned short sizeofInternalData0 = strlen(internalData0);

char internalData1[BUFFER_SIZE];

sprintf(internalData1, "%s", newReceivedBuffer[1]);

unsigned short sizeofInternalData1 = strlen(internalData1);

// -----

switch (internalData1[0]) {

case IDLE_SINGLE_START:

case IDLE_MULTI_END:

communicationFlag = End;

break;

case IDLE_MULTI_START:

communicationFlag = Continue;

break;

default:

communicationFlag = Idle;

// Free up out-of-date buffer data

receivedBuffer[0] = '\0';

sizeofReceivedBuffer = 0;

return false;

}

for (unsigned short index = 0; index < sizeofInternalData0; index++)

receivedBuffer[sizeofReceivedBuffer + index] = internalData0[index];

// Add an endofline character to tail

receivedBuffer[sizeofReceivedBuffer + sizeofInternalData0] = '\0';

sizeofReceivedBuffer += sizeofInternalData0;

// If everything goes well, we will arrive here and return true

return true;

}

bool encodeData(unsigned short sizeofData, char data[]) {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

for (char index = 0; index < sizeofGivenBuffer; index++)

for (char subindex = 0; subindex < BUFFER_SIZE; subindex++)

givenBuffer[index][subindex] = '\0';

sizeofGivenBuffer = 0;

indexofGivenBuffer = 0;

// -----

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Second, calculete size of data and modulus

unsigned short modulusofGivenBuffer = (sizeofData % DIVISOR_NUMBER);

sizeofGivenBuffer = (sizeofData / DIVISOR_NUMBER);

// Add modulos of data, if possible

if (modulusofGivenBuffer > 0)

sizeofGivenBuffer++;

// -----

for (unsigned short index = 0; index < sizeofGivenBuffer; index++) {

unsigned short subIndex;

unsigned short upperBound = (index == sizeofGivenBuffer - 1 ? modulusofGivenBuffer : DIVISOR_NUMBER);

for (subIndex = 0; subIndex < upperBound; subIndex++)

givenBuffer[index][subIndex + 1] = data[(index * DIVISOR_NUMBER) + subIndex];

if (sizeofGivenBuffer == 1)

givenBuffer[index][subIndex + 2] = (char)IDLE_SINGLE_START;

else if (index == sizeofGivenBuffer - 1)

givenBuffer[index][subIndex + 2] = (char)IDLE_MULTI_END;

else

givenBuffer[index][subIndex + 2] = (char)IDLE_MULTI_START;

givenBuffer[index][0] = PROTOCOL_DELIMITERS[0];

givenBuffer[index][subIndex + 1] = PROTOCOL_DELIMITERS[1];

givenBuffer[index][subIndex + 3] = PROTOCOL_DELIMITERS[2];

givenBuffer[index][subIndex + 4] = '\0';

}

// If everything goes well, we will arrive here and return true

return true;

}

// -----

char *generateDelimiterBuffer(char *character, unsigned short sizeofData) {

if (sizeofData == 0)

return false;

// Store count of found on this variable

static char arrayofCharacter[BUFFER_SIZE * MINIMIZED_BUFFER_SIZE];

// Set first bit as end-of-line biti this is for initialize

arrayofCharacter[0] = '\0';

for (unsigned short index = 0; index < sizeofData; index++)

arrayofCharacter[index] = *character;

// Set last bit as end-of-line bit

arrayofCharacter[sizeofData] = '\0';

return arrayofCharacter;

}

unsigned short howManyCharacter(char *character, unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return 0;

// Store count of found on this variable

char countofCharacter = 0;

for (unsigned short index = 0; index < sizeofData; index++)

if (*character == data[index])

countofCharacter++;

return countofCharacter;

}

bool isNumeric(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

for (unsigned short index = 0; index < sizeofData; index++)

if (isdigit(data[index]) == 0)

return false;

return true;

}

bool isAlphanumeric(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Check function ID, type is alphanumeric

for (unsigned short index = 0; index < sizeofData; index++)

if (isalnum(data[index]) == 0)

return false;

return true;

}

## Slave.INO

#include <Wire.h>

#include <Serializer.h>

#include <MemoryFree.h>

// IMPORTANT NOTICE: These all constant is depending on your protocol

// As you can see, this protocol delimiter was declared in this scope

// That's mean, all function will use this delimiter constant on

// Communication between two or more devices

#define DEVICE_BRAND "intelliPWR"

#define DEVICE_MODEL "Slave Tester"

#define DEVICE_VERSION "1.0.0"

// IMPORTANT NOTICE: On I2C bus, You can send up to 32 bits on

// Each transmission. Therefore, if there is more data than 32 bits

// We should split it. Then we can send our data to master

#define DIVISOR_NUMBER 25

// Outside protocol delimiters

#define PROTOCOL_DELIMITERS ""

#define PROTOCOL_DELIMITERS_SIZE 3

// Inside protocol delimiters, we called it data delimiters

#define DATA_DELIMITER ""

#define DATA_DELIMITER_SIZE 1

// Start and end type of protocol delimiters

#define IDLE_SINGLE_START 0x15

#define IDLE_MULTI_START 0x16

#define IDLE_MULTI_END 0x17

// -----

// Do not change default value of this variable

char *receivedBuffer = NULL;

unsigned short sizeofReceivedBuffer = 0;

// Do not change default value of this variable

char** givenBuffer = NULL;

unsigned short sizeofGivenBuffer = 0;

unsigned short indexofGivenBuffer = 0;

const char sizeofFunctionList = 8;

const char *functionList[] = {"getVendors",

"getFunctionList"

};

const char sizeofReturnList = sizeofFunctionList;

const char *returnList[] = {"0",

"0"

};

const char sizeofListenList = sizeofFunctionList;

const char *listenList[] = {"0",

"0"

};

// TEMPORARILY, will be delete on release

#define WIRE_BEGIN 0x60

void setup() {

// Register events, onReceive and onRequest

Wire.onReceive(receiveEvent);

Wire.onRequest(requestEvent);

// Initialize communication on serial protocol

Serial.begin(9600);

// Initialize communication on Wire protocol

Wire.begin(WIRE_BEGIN);

}

void loop() {

}

void receiveEvent(unsigned short sizeofData) {

// Declare variables about given size

char *newReceivedBuffer = (char *) malloc(sizeof (char) * (sizeofData + 1));

unsigned short indexofNewReceivedBuffer = 0;

// Loop through all but the last

while (Wire.available()) {

char currentBuffer = Wire.read();

newReceivedBuffer[indexofNewReceivedBuffer++] = (char)currentBuffer;

if (currentBuffer == (char)PROTOCOL_DELIMITERS[2]) {

newReceivedBuffer[indexofNewReceivedBuffer] = '\0';

break;

}

}

// -----

if (!decodeData(indexofNewReceivedBuffer, newReceivedBuffer))

unknownEvent(indexofNewReceivedBuffer, newReceivedBuffer);

// Do not forget to free up local data

free(newReceivedBuffer);

}

void requestEvent() {

// IMPORTANT NOTICE: On the worst case, we are sending empty protocol

// Data(s) this is the best way of worst case because the receiver device

// Can always Decode this data and when we send "not filled" protocol

// Data(s), this means we are sending empty data(s)

if (indexofGivenBuffer >= sizeofGivenBuffer && sizeofGivenBuffer == 0) {

Wire.write("UNKNOWN_REQUEST");

// IMPORTANT NOTICE: This point can explaine as the least desirable

// Situation. When program arrrives this point, that's mean master

// Send more than calculated data or less than calculated data. But

// Do not worry, I wrote all code blocks as far as stable. It will

// Terminate itself automaticly when it was arrive there

clearGivenBuffer();

}

else

Wire.write(givenBuffer[indexofGivenBuffer++]);

}

// -----

void unknownEvent(unsigned short sizeofData, char data[]) {

// Notify user about available memory

Serial.print("[Available memory: ");

Serial.print(freeMemory());

Serial.print("] ");

// Notify user

Serial.print("Error! Unexpected <");

Serial.print(data);

Serial.print(">[");

Serial.print(sizeofData);

Serial.println("] data received.");

// At the end, free up out-of-date buffer data

clearGivenBuffer();

clearReceivedBuffer();

}

void executeEvent(unsigned short sizeofData, char data[]) {

// Store index of found function

char foundIndex = 0;

bool foundFlag = false;

// Decode given data and store it

char **insideData = Serialization.decode(DATA_DELIMITER_SIZE, DATA_DELIMITER, sizeofData, data);

for (char index = 0; index < sizeofFunctionList; index++) {

// Null operator check

if (insideData == NULL)

break;

// Null operator check

if (insideData[0] == NULL ¦¦ insideData[1] == NULL)

break;

// Calculate registered function length at the given index

unsigned short sizeofInternalFunction = strlen(functionList[index]);

unsigned short sizeofExternalFunction = strlen(insideData[0]);

// Data size check

if (sizeofInternalFunction != sizeofExternalFunction)

continue;

// -----

// Found status flag, using for to find operate

foundFlag = true;

for (unsigned short subIndex = 0; subIndex < sizeofInternalFunction; subIndex++) {

if (insideData[0][subIndex] != functionList[index][subIndex]) {

foundFlag = false;

break;

}

}

// If a function found in function list,

// Store index of this function and break current loop

if (foundFlag) {

foundIndex = index;

break;

}

}

// In the worst case, we can not find a function and found index is -1

if (foundFlag)

callReceivedFunction(foundIndex, strlen(insideData[1]), insideData[1]);

else

unknownEvent(sizeofData, data);

// At the end, free up out-of-date buffer data

clearReceivedBuffer();

}

// -----

bool decodeData(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Decode given data, Calculate up-of-date and needed buffer size

char **newReceivedBuffer = Serialization.decode(PROTOCOL_DELIMITERS_SIZE, PROTOCOL_DELIMITERS, sizeofData, data);

unsigned short sizeofNewReceivedBuffer = strlen(newReceivedBuffer[0]);

// Null operator check

if (newReceivedBuffer == NULL)

return false;

// Null operator check

if (newReceivedBuffer[0] == NULL ¦¦ newReceivedBuffer[1] == NULL)

return false;

// -----

switch (newReceivedBuffer[1][0]) {

case IDLE_SINGLE_START:

case IDLE_MULTI_START:

case IDLE_MULTI_END:

break;

default:

return false;

}

// Malloc and realloc a sentence, a list of words

if (receivedBuffer == NULL)

receivedBuffer = (char *) malloc(sizeof (char) * (sizeofReceivedBuffer + sizeofNewReceivedBuffer + 1));

else

receivedBuffer = (char *) realloc(receivedBuffer, sizeof (char) * (sizeofReceivedBuffer + sizeofNewReceivedBuffer + 1));

for (unsigned short index = 0; index < sizeofNewReceivedBuffer; index++)

receivedBuffer[sizeofReceivedBuffer + index] = newReceivedBuffer[0][index];

// Add an endofline character to tail

receivedBuffer[sizeofReceivedBuffer + sizeofNewReceivedBuffer] = '\0';

sizeofReceivedBuffer += sizeofNewReceivedBuffer;

// -----

// Go to next step, decoding inside data

if (newReceivedBuffer[1][0] == (char)IDLE_SINGLE_START ¦¦ newReceivedBuffer[1][0] == (char)IDLE_MULTI_END)

executeEvent(sizeofReceivedBuffer, receivedBuffer);

// If everything goes well, we will arrive here and return true

return true;

}

bool encodeData(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Second, calculete size of data and modulus

unsigned short modulusofGivenBuffer = (sizeofData % DIVISOR_NUMBER);

sizeofGivenBuffer = (sizeofData / DIVISOR_NUMBER);

// Add modulos of data, if possible

if (modulusofGivenBuffer > 0)

sizeofGivenBuffer++;

// We do not need realloc code because of the top of encoding function

givenBuffer = (char **) malloc(sizeof (char *) * (sizeofGivenBuffer + 1));

// -----

for (unsigned short index = 0; index < sizeofGivenBuffer; index++) {

givenBuffer[index] = (char *) malloc(sizeof (char) * (DIVISOR_NUMBER + PROTOCOL_DELIMITERS_SIZE + 1));

unsigned short subIndex;

unsigned short upperBound = (index == sizeofGivenBuffer - 1 ? modulusofGivenBuffer : DIVISOR_NUMBER);

for (subIndex = 0; subIndex < upperBound; subIndex++)

givenBuffer[index][subIndex + 1] = data[(index * DIVISOR_NUMBER) + subIndex];

// IMPORTANT NOTICE: At the here, We have two status for encoding data(s)

// But if you set this var as multiEND, this means encoding is over

// We are making this for receiver side. singleSTART means that data can encode

// As one packet, do not need any more encoding

if (sizeofGivenBuffer == 1)

givenBuffer[index][subIndex + 2] = (char)IDLE_SINGLE_START;

else if (index == sizeofGivenBuffer - 1)

givenBuffer[index][subIndex + 2] = (char)IDLE_MULTI_END;

else

givenBuffer[index][subIndex + 2] = (char)IDLE_MULTI_START;

givenBuffer[index][0] = PROTOCOL_DELIMITERS[0];

givenBuffer[index][subIndex + 1] = PROTOCOL_DELIMITERS[1];

givenBuffer[index][subIndex + 3] = PROTOCOL_DELIMITERS[2];

givenBuffer[index][subIndex + 4] = '\0';

}

// Do not forget to end-of-line char to tail of 2D array

givenBuffer[sizeofGivenBuffer] = '\0';

// If everything goes well, we will arrive here and return true

return true;

}

// -----

void clearGivenBuffer() {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

for (char index = 0; index < sizeofGivenBuffer; index++)

free(givenBuffer[index]);

// After free up top level pointer

free(givenBuffer);

givenBuffer = NULL;

sizeofGivenBuffer = 0;

indexofGivenBuffer = 0;

}

void clearReceivedBuffer() {

// IMPORTANT NOTICE: Before the calling internal functions,

// Last stored data must be removed on memory. Otherwise, we can not sent

// Last stored data to master device. And additional, data removing will refresh

// the size of data in memory. This is most important thing ...

free(receivedBuffer);

receivedBuffer = NULL;

sizeofReceivedBuffer = 0;

}

bool isNumeric(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

for (unsigned short index = 0; index < sizeofData; index++)

if (isdigit(data[index]) == 0)

return false;

return true;

}

bool isAlphanumeric(unsigned short sizeofData, char data[]) {

if (sizeofData == 0 ¦¦ data == NULL)

return false;

// Check function ID, type is alphanumeric

for (unsigned short index = 0; index < sizeofData; index++)

if (isalnum(data[index]) == 0)

return false;

return true;

}

void callReceivedFunction(unsigned short indexofFunction, unsigned short sizeofData, char data[]) {

// At the first, free up out-of-date buffer data

clearGivenBuffer();

clearReceivedBuffer();

// -----

switch (indexofFunction) {

case 0:

getVendors(sizeofData, data);

break;

case 1:

getFunctionList(sizeofData, data);

break;

default:

break;

}

}

// ------------------------------------------------------------

// ----- INTERNAL FUNC(S) -------------------------------------

// ------------------------------------------------------------

void getVendors(unsigned short sizeofData, char data[]) {

// Notify user about available memory

Serial.print("[Available memory: ");

Serial.print(freeMemory());

Serial.print("] ");

// Notify users

Serial.print("Done! ");

Serial.print(__func__);

Serial.println(" function triggered.");

char *delimiterBuffer = "";

char *resultBuffer[] = {DEVICE_BRAND, DEVICE_MODEL, DEVICE_VERSION};

char *result = Serialization.encode(2, delimiterBuffer, 3, resultBuffer);

encodeData(strlen(result), result);

}

void getFunctionList(unsigned short sizeofData, char data[]) {

// Notify user about available memory

Serial.print("[Available memory: ");

Serial.print(freeMemory());

Serial.print("] ");

// Notify users

Serial.print("Done! ");

Serial.print(__func__);

Serial.println(" function triggered.");

char *delimiterBuffer = "";

char *resultBuffer[] = {"External", "Core"};

char *result = Serialization.encode(1, delimiterBuffer, 2, resultBuffer);

encodeData(strlen(result), result);

}

# ÖZGEÇMİŞ

Ad–Soyad: 	Veysel Berk Altun
Doğum Tarihi ve Yeri: 	03.12.1994, Muş
B.Sc: 	Pamukkale Üniversitesi
E–posta: 	contact@vberkaltun.com

Deneyim:
Haziran 2016 	Kıdemli Yazılım Geliştirici,
 	40idea Laboratory Engineering Lab. & Consultancy Co. Ltd.

Haziran 2012	Servis Teknisyeni,
 	Unipa A.S.

Onur ve Ödüller:
Mart 2013 	ITURO Olimpiyatları Çeyrek Final Ödülü
 	İstanbul Teknik Üniversitesi, İstanbul

Mayıs 2013	“Eğitimlence: eğlenerek öğrenmektir.” Birincilik Ödülü
 	Yaşar Üniversitesi, İzmir

Haziran 2013	Anadolu Teknik Dalında Birincilik Ödülü
 	Mazhar Zorlu Anadolu Teknik Lisesi, İzmir

Mayıs 2018 	“Genç Girişimciler Yeni Fikirler” Birincilik Ödülü
 	Ege Üniversitesi, İzmir